Leveraging State-of-the-art Technologies in the Construction of buildings at Chandigarh

Dhandapani M 1, Vamsi Krishna M 2

1,2 EDRC-B&F IC, Larsen & Toubro Construction, Chennai, India

1kmdpani@lntecc.com, 2vamsi@lntecc.com

ABSTRACT

The two innovative construction technologies that the country is currently experiencing tremendous growth in are explained in this article. They consist of 3D concrete printing and precast construction, each with unique benefits and distinctions. Currently accounting for 10% of the country's growth index, the building industry mostly relies upon expensive labour, formwork, etc. To make the best use of our resources moving forward, we had to evaluate the different possibilities. In this instance, we decided on to implement both 3D printing and precast technology in simultaneously. The project case study focusses on the development of six buildings in Chandigarh that would have transit accommodations and residential space on the ground and two floors. By utilising the adaptability of precast construction and 3D concrete printing technologies, we approached this project as a custom-made, technology-driven design. The entire process—from concept to commissioning—as well as issues with design and execution, will be explained.

Keywords: 3DConcrete printing, Precast construction

INTRODUCTION

The fast urbanisation of India is raising several difficult concerns about the housing of the millions of people condensing into our cities. Under the "Housing for All" initiative, our nation is actively working towards the goal of building 60 million dwellings to meet the expanding demands of urbanisation. It is an enormous task that requires creative, technology-driven solutions to be completed.

In conjunction with the context, L&T Construction, a Builders to Nations and leader in using cutting-edge technologies, started looking at several innovative approaches that could meet the rapid and large-scale demands for mass housing. In the process of finalising the strategic roadmap for the mass housing business, L&T Construction has considered a few technological options and eventually decided that 3D concrete printing (3DCP) and precast construction technologies have the potential to revolutionise the construction sector.

The success of the world's rapid industrialisation may be attributed to automated procedures that result in speedier manufacturing techniques. Precast construction, which uses prefabricated reinforced concrete elements that are manufactured at factory conditions and then installed on-site and have gradually replaced the labour-intensive, time-consuming, and labour-intensive method of mixing & pouring concrete on site over the past three decades, has proven beneficial in certain developed countries, including India. Conversely, the layer-by-layer printing in 3D concrete printing offers benefits like automation, productivity, design freedom, and sustainability and completely avoiding the formwork requirement.

The implementation of 3DCP technology and Precast construction will be demonstrated through a project case study on the construction of 06 buildings with ground+2 levels of residential space and transit accommodation for a reputable client, along with conclusions and future endeavours.

PROJECT BRIEF

One of our esteemed clients has collaborated with L&T to construct their facility campus with the possibility to include the emerging construction technologies.

Construction of a facility campus with the purpose of transit accommodation located at Chandigarh, India, for the defence client.

| Sl.No | Block | Floor Configuration | Length | Breadth | Height |
| --- | --- | --- | --- | --- | --- |
|  |  |  | m | m | m |
| 3D Concrete Printed Buildings |  |  |  |  |  |
| 1 | Block-01 | Ground + 2 Floors | 47.1 | 11.8 | 9.9 |
| 2 | Block-02 |  | 16.3 | 14.3 | 9.9 |
| 3 | Block-03 |  | 17.0 | 14.0 | 9.9 |
| 4 | Block-04 |  | 45.2 | 7.1 | 9.9 |
| 5 | Block-06 |  | 16.5 | 16.8 | 9.9 |
| Precast Technology Building |  |  |  |  |  |
| 1 | Block-05 | Ground + 2 Floors | 43.4 | 48.6 | 9.9 |

B3

B1

The scope of work involves constructing six blocks, of which 50% of the total built-up area will be created using 3D concrete printing and the remaining 50% will be created using precast construction techniques.

The project's uniqueness is showcased through its use of technology without compromising quality, design intent/ shape, or functional needs. The project site effortlessly integrates and implements 3D Concrete printing and Precast construction, the potential outcomes of these two important technologies.

Seismic zone IV & Wind speed 47 m/s as applicable for the location are considered for designs.

TECHNOLOGY 1: 3D CONCRETE PRINTING

3D printing, also known as additive manufacturing, is an automated process that produces complex shaped geometries from a 3D Model (Computer-aided design CAD model) on a layer-by-layer basis. It is used in various manufacturing industries such as automobile, aerospace, medical etc.3D printing in construction is one of the promising ways to automate the construction process which enables the creation of complex designs and shapes with precision and efficiency based on digital models. This will have the potential of saving laborious work, material waste, and construction time.

The construction industry benefits from advanced technology especially for complex designs, which also reduces labour requirements and construction time, improves project quality and reduces environmental impact. risky operation of humans etc. There are two types of 3D Construction printing (3DCP) i.e., onsite and offsite 3DCP. In offsite 3DCP, the units or components are printed in the factory and then transported to the assembly site. In onsite 3DCP, the units are constructed at the site directly.

3DCP DESIGN MIX

A mix of 3DCP recipe (Printable concrete) developed in-house with locally available ingredients such as, cement, fine aggregate, fly ash and chemical admixture, PP Fiber and water. 3D Concrete mix design requires unique combination of concrete properties.

3D Printable fresh state concrete poses following characteristics are very essential to deposit the fresh concrete in layer by layer.

- Workability

- Shape retention

- Bonding

- Buildability

Both the fresh and hardened properties are equally important.

A key characteristic of 3D printable concrete is highly dependent on the fresh state properties and   interdependency of the design detailing, material, process, and geometrical shapes.

STRUCTURAL SYSTEM

All the 3D printing blocks are designed as load bearing reinforced masonry structures. The substructure comprises of cast-in place wall/ strip footing and RC walls up to plinth level. The super structure shall be comprising 3D Concrete printed walls with cast-in place slabs.

CONSTRUCTION METHODLOGY

Layer wise 3D concrete printing

The substructure is to be constructed with the conventional approach i.e. Wall/ strip footing and RC wall connecting to plinth using cast-in place concrete with necessary formwork. For super structure, the erection of 3D Printer in position to, and necessary checks and balances shall be carried out to ensure the coordinates and building dimensions.

We proposed 180mm gross thickness of the 3D concrete printed wall, which comprise of inner and outer ribbon with a gap of 60mm. the ribbon width is 60mm and layer thickness is 25mm with the printing speed of 200 to 250mm/sec.

Wall horizontal reinforcement placed during printing itself as specified in the structural drawings. Printing and rebar placing shall be continuously monitored by skilled workman.

The printing process continue till reaching the slab soffit. Electrical conduit and vertical reinforcements shall be installed in the designated location and grouting the void with flowable 3D Printable concrete mix.

Slab and beam shuttering shall be kept in position, reinforcement and line and level checked before concreting. In-situ concrete poured in the slab and beam moulds.

Gantry type printer

MACHINERY –3D Printer

Now a days, various type of 3D printers are available suitable for the construction,

- Radial / Articulated Printers (Using Robotic Arms)

- Off-site Printing and Complex

- Architectural elements printing

- Cartesian Printers (Gantry Type)

  - Onsite Cast In-situ Printing

  - Buildings Printing

|  | Make | Range |
| --- | --- | --- |
| Printer-1 | Cobod 1 | 11.8 x 11.5 x 8.7m |
| Printer-2 | Merlin | 24.0 x 17.0 x 10.0m |
| Printer-3 | Ajax | 9.0 x 9.0 x 9.0m |

In this project we chose Gantry type printers. The gantry is the frame structure that supports the printer head along the X/Y/Z axis as the printer head moves around to deposit the concrete layers in the designated place.

Three type of large-scale 3D printers are deployed to suit the printing process as per the plan dimensions of blocks, so as the printer can be shifted in one direction for zone wise printing.

Printer-2

Printer-3

Printer-1

3D Concrete printing–Work in-progress

3DConcrete printed blocks

Block-01

Block-03

Block-02 has a unique shape (circular) and every floor staggered to create the unique elegance, which posed a lot of structural challenges to achieve the curved beam connection, in execution and detailing.

TECHNOLOGY 2: PRECAST CONSTRUCTION

Precast construction technology is a system of casting concrete in a reusable mould which is then treated in a controlled environment and transported to the construction site and then lifted to the place on-site. Precast construction system consists of various precast elements such as walls, columns, beams, slabs, staircase, and some customized elements that are standardized and designed for stability, durability and structural integrity of the building. Precast residential building construction involves design, strategic yard planning, lifting, handling and transportation of precast elements. This technology is suitable for the construction of buildings resisting seismic and wind induced lateral loads along with gravity loads, even for high-rise construction. The framing of the building should be planned to achieve the maximum number of mould repetitions. These elements are cast in a controlled factory condition. The factory is developed at or near the site which provides an economical solution in terms of storage and transportation; in some circumstances, this factory may be vendor-based.

Block-05: Structural framing

Block-05

This block comprises of Ground + 2 floors building with total built-up area of 50,000 Sq.ft. This is proposed to have the transit accommodation at 1st& 2nd floors whereas the Ground floor is planned to have the multipurpose rooms.

This block is proposed as the largest in horizontal coverage with plan area of 48.6 x 43.4m in the project.3D printing is not possible due to the limitation of printer’s reach, since the plan dimensions are beyond the limit of printer maximum width.

By considering the timelines to deliver the project, the alternate system has been chosen as the Precast construction technology, so that the construction can be simultaneously with other blocks using 3D printing.

STRUCTURAL SYSTEM

The structural system comprises of dual system with precast column & beam frame system with hollow core slabs as floor slab spanning between the beams and precast shear walls at core areas. Foundation system is of cast-in place Isolated/ combined footings.

Part plan showing structural system Structural framing

The columns, beams and hollow core slabs are designed to take care of the gravity loads. The positioning of the columns is arranged in a pattern to suit the architecture intent and effective utilization of the hollow core slabs. The building has external façade all-round the periphery which are also precast walls.

An effective lateral load resisting system with precast shear walls proposed at Staircase core areas. These shear walls are designed to resist 100% of lateral load.

Moment connections are proposed at beam column junctions by reinforcement continuity and stitch joints of the beam column junctions.

The lateral shear shall be transferred to the shear walls through the rigid diaphragm action. The diaphragm action of the floor is achieved through the system of structural topping and the structural integrity is achieved through the connections.

SALIENT FEATURES

Precast columns from foundation

Precast Construction is implemented from top of the foundation, by considering the advantage of minimum founding level due to hard soil strata available.This building entirely comprises of precast elements (structural & non-structural including partition walls) of around 1770 precast elements along with architectural finishes and MEP works.

- Precast from top of foundation

Precast columns are bifurcated floor wise, and the ground floor elements are designed with extra height which are supported over the foundation.

To provide suitable support for the adjacent beam elements, these columns are pre-casted up to the beam soffit and are featured multi-direction corbels. The beam-column junction will then be completed on site using stitch concrete.

- Hollow core slab system

Hollow core slabs erection

Prestressed precast Hollow core slabs (HCS) are designed for the longer floor span 10.0m by considering the appropriate loading and hence derived the suitable section of the slab as 0.25m thick. The reinforcing strands were optimized for the lesser spans of hollow core slabs.

This adoption had resulted to eliminate the other direction tie beams & secondary beams which in turn reduced the activities & floor cycle time.

These elements are produced typically 1.20m wide & the length as per requirement (*panels cut from the bed at factory).Predefined sleeves/ cutouts were incorporated in the HCS elements after having the detailed coordination with multi-disciplinary services routing.

Precast facade at elevation

Precast façade

External walls which are load bearing as part of façade in elevation. Joints are designed to ensure water tightness with the utmost priority. A nib or step-like profile is created in the wall panels located at the periphery, which are exposed to rain, so that water movement can be restricted in an upward nib profile from the outside.

Nib profile can be projecting outwards or within wall thickness to prevent water ingress while maintaining architectural elevation and aesthetic appearance.

CONNECTIONS

Connections are needed not only to transfer loads but also to provide continuity and ensure the monolithic behaviour of the entire structure. They are designed with the emulative detailing concept, so that once completed, will behave like an equivalent RCC System, providing the necessary strength and ductility.

Emulative detailing is defined as designing a connection system in a precast concrete structure such that its structural performance is equivalent to that of a conventionally designed, cast-in-place, monolithic concrete structure. In horizontal and vertical joints, concrete is made continuous by filling the joint with non-shrink non-metallic, high-strength grout, or by casting with stitch concrete.

Groutec couplers are used as mechanical splices for precast column to column connections, which makes erection fast and simple.

The seismic diaphragm action is ensured by the composite action of structural topping of 75mm with the hollow core slab and the structural integrity ensured between structural topping & the beams. The structural diaphragm ensures the transfer of lateral force to the lateral resisting system and the design has been carried out to study the behavior of diaphragm and designed/ detailed to withstand the chord forces.

Precast factory

PRODUCTION

The block-5 structure is involved with the production & installation of around 1770 precast elements of different types.

Due to the project's extremely constrained precast scope, we have chosen to have the elements produced by a vendor and delivered to the installation site.

Hollow core slabs

For the production and supply of the elements, M/s. Info Stellar, a precast firm in Delhi, was chosen. According to the specifications of the design, this factory produces any kind of element like columns, beams, walls, slabs, stairs, hollow core slabs, 3D elements, etc., and gets them delivered to the construction site.

Block-5: Precast Construction

CONCLUSION

Even though this is a technology-driven project, the 3D concrete printed buildings and the Precast building have been completed in accordance with the intended architectural intent with higher precision, improved efficiency, and reduced waste.

In this country, 3D concrete printing technology is still in its infancy. However, the Building Materials and Technology Promotion Council (BMPTC) has approved its implementation for Ground + 3 floors buildings. But the printers that are now available can support printing up to Ground +2 floors.

The material that is used in 3D printing must meet certain requirements. Firstly, it should be flexible and able to be pumped; once extruded, and it should be able to withstand the shape without the need for formwork. Secondly, it should possess certain properties when it begins to harden, such as load bearing and how the layers will bond. There is still much to learn about this rapidly evolving field of technology, and new tests are required because the material must possess certain qualities to be considered appropriate.

The future will explore the large-scale printing of prefabricated 3D concrete panels at the factory and their on-site erection and connecting. By doing this, the printer limitations for the horizontal reach will be avoided, and the predetermined printer deployment at the factory may move forward.

ACKNOWLEDGEMENT

The authors of the paper would like to specially acknowledge all the project's stakeholders for their incredible efforts and contributions, and the statutory/ governing bodies, as well as the management of Larsen & Toubro Construction, Buildings & Factories IC.