| Harsh Parwani AGM -Engineering Kalpataru Project International Limited, Mumbai Harsh Parwani, born 1986 received his civil engineering Degree From University of Mumbai and having a total 10+ year Experience in the field of Bridge engineering. harsh.parwani@kalpataruprojects.com | Rakesh Kumar Mehta AVP-Engineering Kalpataru Project International Limited, Mumbai Rakesh K Mehta, born 1972 received his Master degree From University of Roorkee. He has 26 Years of Experience in Bridge Design & Construction method for Infra project. rakeshkumar.mehta@kalapataruprojects.com |
| --- | --- |

DESIGN & CONSTRUCTION OF PRE TENSIONED U BEAM IN HIGHWAY STRUCTURE - ETHIOPIA EXPRESSWAY

ABSTRACT

In earlier days, the majority of superstructure of large width bridges, were usually designed with pre-stressed I girder superstructure. This was widely followed since the cost of setup, or for the matter of fact, the cost of erection was less. Moreover, erection of I girders was an easier job.

Breakthroughs were made in the field of segmental box construction since the early onset of the last decade. Slowly the design and construction industry started to innovate with box segmental construction to construct wide bridges overcoming the above mentioned aspects

However, there were still many areas/regions where this segmental construction will not work, like small and intermittent viaducts, overpasses, highway projects. In such locations, I girder construction is still being followed without much innovation and seems the most feasible solution. These I girder designs were either RCC or PSC girders (primarily post tensioned). Although industry slowly and gradually adopted the pre-tension I girder deigns, which brought some consistency, uniformity as well as a device for mass production, still aesthetics, long term maintenance and attractiveness were far from achieved.

To achieve the desired aesthetics, yet maintain the economy of precast, pre-stressed girders manufactured under controlled plant conditions, a new shape was developed. The result is the U-beam, a trapezoidal open-top section with sloping webs. Although the concept was originally designed in the west, but very reluctantly and rarely adopted in Indian conditions, except the patented Metro U girders.

This U girder or the U beam girder is now successfully adopted and followed in one of our highway projects in Ethiopia. This superstructure was designed as pre-tensioned U shaped girders with deck slab. The only driving force behind the design was to ditch the orthodox designs and bring some innovations. Moreover, aesthetics was an added benefit with this shape of the superstructure, along with speed and cost factors.

Keywords: I girder, Economy, pre-tension, innovation, aesthetics, superstructure, U girder

INTRODUCTION

Ethiopian Road Authority (ERA) has undertaken the construction of Rs 1085 Crore Four lane expressway, along Adama Awash corridor, in Ethiopia. This project was awarded to M/s Kalpataru Projects India Limited (Erstwhile JMC Projects) on EPC basis. This expressway will provide an unhindered movement of goods and traffic between Adama and Awash and further up to the border of Ethiopia and Djibouti.

The proposed project road connects Adama and Awash located in the eastern part of Ethiopia under the administration of three Regional States, Oromia, Afar and Amhara. The start point is Adama which is about 90 km away from the capital city, Addis Ababa along the existing Addis Adama toll road. The existing alignment of project road stretches around 125 km starting from Adama through Wellenchiti, Metehara and ends at Awash. The project road is part of the road segment from Addis Ababa to Djibouti which has the highest traffic volume and load in the country as it is the main corridor of port Djibouti. It is an extension of Addis Ababa-Adama Toll road which expressway is completed and under full service now.

Fig:1. Key Plan of Project

BRIEF GEOLOGICAL DESCRIPTION OF THE LOCATION:

Fig: 2.Geological Map of the Proposed Project Alignment

This Expressway is being constructed in the Main Ethiopian Rift (MER), where active continental rifting creates specific conditions for geohazards. The rift is basically a peculiar site affected by active tectonics; weak and unstable geological formations; active and sensitive geomorphology; within semi-arid climate conditions. Structural and morphotectonic evidences support that it is defined by an active extensional tectonics and the intense volcanism associated (Agostini et al., 2011). It then related to frequent hazardous geological phenomena. Endogenous risk factors such as earthquakes, volcanism, and post-volcanic phenomena are closely related with tectonics in this area. The geomorphology of this expressway is also highly variable across the MER and is mainly the result of volcanic and tectonic events with the associated erosional and depositional processes.

The main Ethiopian rift is one of the active and complex geological site. It was a challenge to accurately define the possible geo hazards related to these rift processes at this location. The risks associated to these geo hazards are also uncertain and pose definite constrains to effectively design and construction of this expressway.

BRIEF DESCRIPTION OF THE PROJECT:

The total length of the expressway is 60 km. Apart from the at grade road, it mainly consist of 22 Vehicular Overpasses, among other small structures. The span length of overpasses varied from 20m to 30m. these overpasses were built to carry the cross traffic across the expressway.

SALIENT FEATURES OF THE PROJECT:

- Start Date – 				12/4/2018

- Client - 				Ethiopian Roads Authority.

- Contractor – 				Kalpataru Projects International Limited.

- Location - 				Adama, Ethiopia

SALIENT FEATURES OF THE VEHICULAR OVERPASS:

- Span of Overpasses – 			20m+20m

- Total Number of Overpasses – 		22

- Type of Foundation – 			Open Foundation.

- Pier and Abutment - 			Rectangular Pier with flared Pier Cap.

- Type of Superstructure - 		Precast Pre-stress U Girder.

Fig: 3. Typical GAD of a Vehicular Overpass

Some of the salient design parameters considered in the design are as below:

- Seismicity

- Seismic zone				: IV

- Seismic coefficient			: 0115G

- Permissible increase in SBC		: 25%

- Permissible increase in stress		: As per AASHTO LRFD

- Soil parameter

- Density (dry)				: 1.8 t/m3

- Density (angle of internal friction)	: 25 degree

- δ  (Friction between soil and face)	: 15 degree

- SBC for open foundation		: 28 t/m2 gross at founding level.

- Material

- For condition of exposure		: Moderate

Concrete grades

- For pretension girders			: C40

- For OPEN foundation			: C30

- Reinforcements 			: HYSD bars conforming to IS:1786

- Miscellaneous

- Type of bearings			: POT and POT cum PTFE

- Wearing coat				: 50 mm thick  bituminous concrete

- Expansion joint				: Slab seal expansion joint

- Software package			: STAAD III - Release 22.0

- Super Structure Features

- Span – 					: 23.18m

- Girders/ span – 				: 3

- Deck Width -				: 10m

- Girder Width – 				: 2.15m.

- Girder Weight – 			: 36t/girder

- Pre stressing – 				: Longitudinal (Pre-Tensioning)

- Concrete Grade of Girders -		: C40

- Concrete strength at transfer- 		: 31mpa

- Deck casting after girder - 		: 28 days

- Loading - 				: AASHTO LRFD

- Strand Area - 				: 140 mm2

- Breaking Strength -			: 260 kn

- Characteristic Strength -		: 1860 mpa

- Jacking Stress -				: 0.75 UTS

SUPERSTRUCTURE:

Superstructure for this overpass was conceptualized and finally designed as ‘pre-stressed, pre-tensioned ‘U Girder’ to make the design most optimum with respect to weight, concrete quantity, and substructure designs. 3 number of U Girders were provided next to each other in the entire width of the overpass.

Fig: 4. Typical Section at Abutment

Fig: 5. Typical Section at Pier

Fig: 6. Typical Detail of U Girder

INNOVATION & INSPIRATION:

An innovative superstructure design was proposed and adopted for this particular overpass structure. Instead of going with conventional box girder or I girder, it was conceptualized and finally designed as ‘pre-stressed, pre-tensioned U girder. This new design helped in overall reduction of around 15% against the conventional system.

Superstructure for this overpass was conceptualized and inspired from Texas U girder beam. As it can be seen from image below how a ‘U girder’ can be a substitute for 2 numbers of I Girder.

In this case, among the various options available like I girder and solid slab, box girder finally the competition was between post-tensioned I girder versus pre-tensioned U beams. In this particular context, pre-tensioned beams had certain advantages and also peculiarities vis-à-vis its counterpart.

The inherent peculiarities such as single-stage pre-stressing, transfer of pre-stress through bond between concrete and cables by obviation of grouting, sheathing ducts, tensioning of tendons before the concrete is cast and transfer of pre-stress after the attainment of required strength in concrete, warrants specially designed casting bed which should be capable of imparting required quantum of pre-stressing force.  These peculiarities derive certain advantages in favour of pre-tensioned beams in terms of durability, quantity reduction, construction speed, and design and construction expediency.

As the pre-tensioned girders are provided in factory like environment where the bonding between concrete and tendons is direct due to the absence of grouting inside the sheathing duct, the better durability and corrosion resistance is achieved.  The absence of the cables in the web and the elimination of end blocks and blisters to house the anchorages, allows the section optimisation from strength criteria alone.  This helps in reducing concrete quantities rendering in lighter beams, facilitating in attenuation in cost of handling, transportation and erection. Most importantly, the total number of girders in U beam pretension girders was 1 less than I girder system.

Fig: 7. PT Strand Arrangement

The value engineering carried out during conceptualization stage for overpass revealed that for the same span of 23m and 3 number of U beams on the cross section, the quantities for pre-tensioned girders are substantially lesser than post-tensioned beams due to its inherent shape. The U girders were suitably integrated with the deck slab to provide the riding surface.

Fig: 8. Typical Details for Deck Slab Integrated With U Girder

Stages of Analysis

- For the analysis in stage 1 only girders are considered with the load of deck slab

- In stage 2 analysis composite property is considered of the U girder + deck slab + LL.

The thickness of the bottom flange and side webs was kept bare minimum in order to provide the most optimum and lightest section. The Typical U beam shown above is 1280 mm deep and

is an aesthetic alternative to normal construction. As known, the I shaped beams have lighter sections, but because they are more closely spaced than U-beams, 1.7 to 2.0 times as many I-shaped beams are typically required per span. Therefore, the total weight of a U-beam span is usually less because the reduced number of U-beams more than offsets the additional beam weight, particularly with this. 1280 mm beams. A secondary benefit of the reduced superstructure dead weight is a reduction in substructure, with fewer and/or smaller columns required. The foundation requirements for the total structure are likewise reduced. In addition to these economic advantages, the cleaner visual lines enhance the appearance of the total structure. The carriageway was designed for following forces.

A maximum of 32 strands in the typical U beams are be placed in the lower flange and webs. Strands are straight and are debonded in the end regions as required. The U-beams are typically designed to the same debonding criteria as used in AASHTO LRFD. In this case the number of partially debonded strands does not exceed 25 percent of the total number of strands. The number of debonded strands in any horizontal row does not exceed 40 percent of the strands in that row. Not more than 40 percent of the debonded strands, or four strands, whichever is greater, shall have the debonding terminated at any section. Debonded strands have been symmetrically distributed about the center-line of the member. Debonded lengths of pairs of strands that are symmetrically positioned about the center-line of the member shall be equal. This Debonding criteria can be easily seen from the table and diagram below.

Fig: 9. Half Sectional Plan Showing Strands Profile

The section and the PT profile and arrangement was provided just in order to have the most optimal designs. Also, all the AASHTO provisions were fully utilized. Following were the stress limitations adopted with respect to the grade of concrete adopted. These are fully synced with the guidelines given in AASHTO LRFD. In our case the maximum tensile stress were limited to 3.1 mpa. And the maximum compressive stress were limited to 12 mpa for the service stage

The percentage loss of stresses experienced in our design was a maximum of 24%.

Photo:1. Showing U Girders in Erected Position

The deck slab was cast over precast RCC sacrificial panels so as to avoid any shuttering as site. Moreover to achieve the desired economy, the planks were integrated with the deck slab to economize the deck slab design. U girders were left with the diaphragm reinforcement during casting and finally they were integrated with deck slab and girder to achieve the composite action.

Fig: 10. RCC Details of Deck Slab

HANDLING AND ERECTION:

The lateral stability of beams including the adequacy during the handling and erection stage was ensured by extensive calculations and checks. The stresses in sections of the beam during the erection stage were ensured to be within the permissible limits as specified AASHTO LRFD.

PRE-TENSIONING AT YARD:

The key factors in the choice and capacity of pre-tensioning bed was the availability of time for pre casting girders and the economic considerations. The cost benefit analysis for various capacities of long line pre tensioning bed was carried out.  Since the expenditure on two beams casting was found to be economically optimum, considering the project duration, the bed for casting two beams was chosen, making the pre-tensioning bench of length of 48m, for stressing 32 number of strands.

Fig: 11. Pre Tensioning Beds for U Girders

Photo:3 . Showing U Girders Stacking Bed

CONCLUSION:

One goal for U-beam construction was to minimize changes to usual construction methods. Precast concrete panels span between 1-shaped beams, and also between and over the U-beams. The same applies to permanent metal deck forms, should the contractor select that deck-forming method. There is no difference in deck construction between a pre stressed concrete 1-shaped beam span and a U beam span except for the forming of the sloping overhang. Even for the overhang detail, the brackets required are similar to those required for conventional construction.

U-beam bridges are anticipated to be competitive in cost with pre stressed concrete 1-shaped beam bridges. Prior to the first U-beam project, which was let in March 1993, an estimate of beam cost was made and it is found to be around 15% cheaper in considering the total built in costs.