Study of Wind Force on Cable-Stayed Bridges with Varied Girder Configuration and Fairing Integration

Nishi Gupta1 (0000-0001-9062-7090), Vipul Prakash2 , Rakesh Ranjan3

1 Research Scholar, IIT Roorkee, Uttarakhand, India

2 Professor, IIT Roorkee, Uttarakhand, India

3 Assistant Professor, IIT Roorkee, Uttarakhand, India

nishi_g@ce.iitr.ac.in

Abstract. Cable-stayed bridges have become increasingly popular in modern infrastructure due to their aesthetic appeal, structural efficiency, and cost-effectiveness. However, their susceptibility to wind-induced vibrations poses a critical challenge in ensuring their long-term structural integrity and safety. This study investigates how altering the girder shape, such as changing its cross-sectional profile, influences the flow patterns and aerodynamic forces acting on the structure. Additionally, the effectiveness of fairings, placed strategically along the girder, in mitigating wind-induced vibration is assessed. The aerodynamic forces and structural responses of cable-stayed bridges with different girder configurations were studied through experiments in the wind tunnel laboratory and through numerical analysis in Ansys Fluent software. Various parameters were considered for the analysis, such as the angle of attack, wind speed, and shape of the girder. The experimental setup incorporated a scaled-down model of the bridge girder. Uniform wind flow field under five attack angles of -5°, -4°, -3°, -2°, -1°, 0°, +1°, +2°, +3°, +4°, and +5° were considered during wind tunnel tests. Three velocities on the section model were considered for the experiments, i.e., 7 m/s, 8 m/s, and 9 m/s. The section model was built on a scale of 1:50 of the prototype. The acquired wind tunnel data were used to determine the bridge's aerodynamic coefficients, such as lift, drag, and moment coefficients. These coefficients are crucial for understanding the wind-induced forces and moments. The findings provide valuable insights into the aerodynamic performance of cable-stayed bridges and are useful for optimizing their structural robustness and wind resistance.

Keywords: Static wind effect, Aerodynamic coefficients, Fairings, Wind tunnel test, Cable-stayed bridge

1 Introduction

With the rapid development of transportation infrastructure in the world, more and more long span bridges crossing mountains, rivers, and lakes are under construction and are being proposed [1]. These long-span bridges with low natural frequencies are very sensitive to wind actions. The aerodynamic and aeroelastic behavior of long-span bridges is widely recognized as one of the fundamental problems that affect the study and the construction (preliminary and final design, building, and operational life) of such structures [2]. The last two decades have shown tremendous advancements in the field of wind tunnel testing, analysis, computation, and construction techniques. Interest in understanding the dynamic behavior of bridges and developing and testing alternative designs has grown significantly since the field emerged in response to the Tacoma Narrows Bridge collapse. [3].

Cable-stayed bridges have inherently greater stiffness (in comparison to suspension bridge) imparted by the self-anchored inclined stay cables [4]. These present a magnificent appearance owing to the varieties of structural forms of pylons and cable patterns, making cable-stayed bridges more attractive compared to suspension bridges in the overlapping span range. The advanced techniques have helped constructing cable-stayed bridges of main span over 1000 m (Stonecutters bridge of 1018 m planned in Hong Kong, Sutong bridge of 1088 m planned in China) after 890 m span Tatara Bridge in Japan opened in 1999 [5]. The design of a long-span cable-stayed bridge is mostly dictated by the aerodynamics of the deck, cables, and pylons [6]. Nevertheless, Research is continuing in an effort to make spans even longer by meeting the aerodynamic challenges of long span cable stayed bridges. Fujino and Nagai [7] have prophesied that cable-stayed bridges of main spans upto 1400 m would be economical.

Although consensus pervades many areas of bridge aerodynamics, it remained a subject littered with varying findings and opinions from those immersed in it. Many of the mechanisms involved- flutter, buffeing, turbulence, galloping and vortex shedding still cause debates amongst different researchers regarding individual or combined roles of these phenomena [8]. Even some terminologies have persistent confusion. Relative roles of full aeroelastic models, section models, computer simulation, mathematical analysis, and modes to be included continue to evoke debate [9].

Chen et al. [10] performed wind tunnel test and found that optimizing the shape of the pylons and deck could further reduce wind loads. The noted that streamlined pylons with aerodynamic profiles minimized the impact of turbulent wind forces, contributing to improved overall performance. Simiu and Scalan [11] discussed the influence of stay cables design on wind performance. They found that optimizing the arrangement and tension of stay cables could mitigate adverse aerodynamic effects and improved the bridge’s response to wind loads. Lee and Kim [12] focused on the design of pylons, noting that modifications such as increasing pylon height and altering cross-sectional shapes could enhance aerodynamic stability. Their study showed that carefully designed pylons reduced wind-induced forces and improved overall structural performance. Li et al. [13] utilized CFD simulations to analyze the wind interaction with cable-stayed bridges. Their findings highlighted that CFD could accurately predict wind induced forces and provide guidance on design modifications to optimize performance. Baker et al. [14] conducted comprehensive wind tunnel tests on cable-stayed bridge models to assess the impact of various design features on wind performance. Their research underscored the importance of testing different configurations to identify the most effective solutions for enhancing wind resilience. M. Keerthana et al. [15] have done wind tunnel investigations on aerodynamics of a 2:1 rectangular section for various angles of wind incidence under uniform and smooth flow conditions. Variation of mean drag and lift coefficients, Strouhal number with angles of wind incidence have been studied.

Enhancing the wind performance of cable-stayed bridges involves a multifaceted approach, including aerodynamic improvements, structural modifications, and consideration of environmental factors. Advances in both computational and experimental methodologies have provided valuable insights into optimizing bridge designs for wind resilience. Future research should continue to explore innovative design solutions and control systems to further improve the performance and safety of cable-stayed bridges under varying wind conditions.

Cable-stayed bridges are renowned for their aesthetic appeal and structural efficiency. However, their performance under wind loads is critical for ensuring safety and longevity [9]. This research investigates the optimization of the wind performance in cable-stayed bridges, focusing on aerodynamic enhancements, structural modifications, and design considerations. In the wind resistance analysis, static coefficients are the key parameters for analyzing bridges' aerostatic instability, buffeting response, and galloping stability [16]. Thus, Accurate assessment of static coefficients is crucial for designing long-span bridges to withstand wind forces. Currently, static coefficients of bridge deck sections are mainly measured using global force and pressure distribution methods in wind tunnel experiments. For the global force method, force balance is used to measure the static wind loads. In the pressure distribution method, pressure can be obtained on each local region by placing pressure taps on the cross-section, and then the static wind loads of the cross-section can be obtained by integrating the wind pressures.

In the present paper, the steady aerodynamics of the bridge sections have been studied in the wind tunnel lab to compute the classical aerodynamic coefficients (Cd, and Cm) for streamlined deck, bluff deck and faired decks (100% fairing, and 60% fairing).

  - Aerodynamic Coefficients

The forces per unit length Fx and Fy along the fixed body axes (x-axis and y-axis respectively) are computed using the measured pressures. The mean force coefficients in x, y directions are obtained as given below [17]:

CFx= FxB12ρUz2   and  CFy= FyB12ρUz2

where, CFx, CFy = mean force coefficient along x and y axes and B is the characteristic dimension which is taken as the dimension normal to the flow for the present case. The aerodynamic forces acting on a structure exposed to wind can be resolved into drag force FD​, which acts in the direction of the mean wind, and lift force FL​, which acts perpendicular to the mean wind direction. By breaking down the forces Fx​ and Fy​ into components aligned with and perpendicular to the wind direction, the drag force FD​ and lift force FL​, respectively are obtained. Forces along drag and lift directions are computed based on the equations given below:

CD= FDH12ρUz2   and  CL= FLB12ρUz2

The wind induced moments are the effects resulting from the normal forces on the bridge surface at top and bottom multiplied by their lever arms and integrated over the entire surface. Moment coefficient is computed from [18]:

CM= FDB212ρUz2

where,

CD, CL, CM = mean drag, lift force and moment coefficients, respectively

H = Height of the bridge section

B = Width of the bridge section

12ρUz2 = reference pressure at deck height, z, due to mean wind speed, U where ρ is the density of air

2 Methodology

2.1 Instrumentation

Experimental methods provide a feasible way to determine static aeroelastic forces such as Drag, Lift, and moment. The flow generating system used in the present study to test the bridge section models is the open circuit type boundary layer wind tunnel at the Indian Institute of Technology Roorkee. The tunnel has a test cross-section of 2.0 m x 1.85 m at the inlet end and varies uniformly to 2.0 m x 2.10 m at the section of testing. A schematic line diagram of the wind tunnel and its salient features are shown in Fig. 1 and Fig. 2. It has become possible to set the fan speed at any specified RPM to maintain a constant wind speed accurately throughout the test. Under normal circumstances, without any surface roughening devices, the wind tunnel generates a 200 mm thick boundary layer with fairly smooth flow (turbulence intensity 0.5% to 1.0%). As the wind tunnel had provisions for testing only the floor-mounted models like buildings and towers, it required a few modifications to fix and test the different section models. Any possibility of non-uniformity of flow near the vicinity of the model on this account is ruled out. A suitable boundary condition was provided and fabricated to be fitted to the load cell.

Table 1. Salient Features of Wind Tunnel Lab at IIT Roorkee

| Total Length of wind tunnel | 38 m |
| --- | --- |
| Length of test section | 15 m |
| Length of Diffuser | 16 m |
| Cross-section details of Test Section | 2 m×1.85 m (Upstream End) |
|  | 2.10 m×2.15m (Downstream End) |
| Turn Table Diameter | 1.8 m |
| Maximum Wind Speed | 16 m/s |
| Capacity of Blower (Fan) | 75 m3/s |
| Power of Motor | 125 H.P. |
| Speed of Motor | 1200 RPM |

Fig. 1. Schematic diagram of open wind tunnel at IIT Roorkee with elevation view

Fig. 2. Schematic diagram of open wind tunnel at IIT Roorkee with plan view

Velocity above floor was measured using Testo-480 instrument. Five Component Load Cell was used to measure the base shear (Fx and Fy) base moments (Mx and My) and twisting moment (Mz). Typical geometric length scales range are between 1:10 and 1:100 Considering the blockage effect and the order of length scale of turbulence expected in the present study, a geometric length scale of 1:50 was selected i.e.,

Lr=LpLm=50

The kinematic scaling of the model of bridge decks is chosen according to Froude’s number

i.e.,

Vr=Lr=50=7.07

2.2 Fabrication of Section Model

MODEL 1 (Streamlined Section)

- Span of the model = 800 mm

- Width of model = 670 mm.

Fig. 3. Cross-sectional details of model 1

Fig. 4. Model 1 fabricated in wind tunnel lab

MODEL 2 (Signature Bridge Model)

- Span of the model = 800 mm

- Width of model = 680 mm

Fig. 5. Cross-sectional details of model 2

Fig. 6. Model 2 fabricated in wind tunnel lab

MODEL 3 (100% Fairing on Signature Bridge Model)

To study the effect of fairings on the original girder deck two modified section models were subsequently tested with attaching fairings on model 2. Different percentage levels of fairings attached at the edges.

- Span of the model = 800 mm

•     Width of model = 750 mm

Fig. 7. Cross-sectional details of model 3

Fig. 8. Side View of Model 3                                                                                 Fig. 9. Model 3 fabricated in wind tunnel lab

Model 4 (60% Fairing on Signature Bridge Model)

Model-III represents Partial faired model of the modified section models with 60% fairings on both the edges of the model 2. The placement of the partial fairings have been done symmetrically.

- Span of the model = 800 mm

- Width of model = 750 mm

Fig. 10. Cross-sectional details of model 4

Fig. 11. Side view of model 4                                                                 Fig. 12. Model 4 fabricated in wind tunnel lab

2.2 Velocity Measurement

First of all, wind velocity profile was measured using velocity probe at the location of load cell (where model is to be placed later) without the presence of the model, but with the presence of vortex. Velocity profiles were measured corresponding to three different speeds of the motor namely 300 RPM, 400 RPM, and 500 RPM With the help of velocity profile shown below boundary layer at the vicinity of load cell was measured. Height for mounting the models was decided as per the boundary layer thickness.

Figure 13 shows that the depth of the boundary layer is approximately 200 mm. Therefore, the model has been suspended at a height of 350 mm to ensure a constant velocity as the height increases.

Fig. 13. Boundary layer profile on the load cell

2.3 Force Measurement (Calibration of load cell)

After placing the model on the load cell, known amount of horizontal load (Fx) was applied at the top of girder in x-direction in steps which caused certain value of moment (My) at the center of the load cell. Calibration coefficient for Fx was obtained by plotting reading on the display unit for Fx Vs applied load (Fx). Similarly, plot was prepared using reading on the display unit for My Vs moment developed (My) for getting calibration coefficient for My. Thereafter, known value of horizontal load (Fy) was applied at the top of girder in Y­direction in steps to obtain the calibration coefficients for Fy and Mx.

Fig. 14 to Fig. 17 enlist the values of drag force (Fx) and moment (My) observed on the display unit when model 1, model 2, model 3, and model 4 are placed on the load cell with Terrain Category I. The test runs were repeated for various conditions for each bridge, and the average values of drag force and moments were calculated.

Model 1 (Streamlined Deck)

Fig. 14.  Calibration plot for Fx (Drag Force) and My (Moment) for Model 1

Model 2 (Bluff Deck)

Fig. 15. Calibration plot for Fx (Drag Force) and My (Moment) for Model 2

Model 3 (100% Faired Deck)

Fig. 16. Calibration plot for Fx (Drag Force) and My (Moment) for Model 3

Model 4 (60% Faired Deck)

Fig. 17. Calibration plot for Fx (Drag Force) and My (Moment) for Model 4

3 Result and Discussion

To study the effect of fairings on an existing bridge in an attempt to increase its stability, suitable fairings were designed and fabricated. The results of Static aerodynamics coefficients are discussed here. The magnitudes of drag force (N) and moment (N-m) acting on Models 1 to Model 4 were measured with respect to changes in motor speed (300 RPM, 400 RPM, and 500 RPM corresponding to wind velocity of 6.8 m/s, 8.9 m/s, and to 11 m/s respectively) for the angle of attack ranging from -5° to +5°. These values were obtained using calibration equations and drag and moment coefficients were then calculated using steady aerodynamic equations. The graphs (Fig. 18 to Fig. 23) were plotted to represent these results.

For 300 RPM

Fig. 18. Variation of Drag Coefficient w.r.t. angle of attack

In Fig. 16, the drag coefficient for the streamlined deck is the lowest among all the decks, while the drag coefficient for the bluff deck is the highest. Providing 100% fairing on the bluff deck is advantageous. The graph shows that 60% fairing falls within the range between the bluff deck and the fully faired deck. For a 0° angle of attack, the drag coefficient is the lowest, and it increases as the angle of attack increases for both negative and positive angles.

Fig. 19. Variation of Moment coefficient w.r.t. angle of attack

In Fig. 17, the moment coefficients for the streamlined deck is the lowest among all the decks, while the drag coefficient for the bluff deck is the highest. Providing 100% fairing on the bluff deck is advantageous. The moment coefficient is lower at a -5° angle of attack and increases as the angle of attack ranges from -4° to +5°.

For 400 RPM

Fig. 20. Variation of Drag Coefficient w.r.t. angle of attack

In Fig. 18, the drag coefficient for the streamlined deck is the lowest among all the decks, while the drag coefficient for the bluff deck is the highest. Providing 100% fairing on the bluff deck is advantageous. The graph shows that 60% fairing falls within the range between the bluff deck and the fully faired deck. For a 0° angle of attack, the drag coefficient is the lowest, and it increases as the angle of attack increases for both negative and positive angles.

Fig. 21. Variation of Moment coefficient w.r.t. angle of attack

In Fig. 19, the moment coefficient for the streamlined deck is the lowest among all the decks, while the drag coefficient for the bluff deck is the highest. Providing 100% fairing on the bluff deck is advantageous. The moment coefficient is lower at a -5° angle of attack and increases as the angle of attack ranges from -4° to +5°.

For 500 RPM

Fig. 22. Variation of Drag Coefficient w.r.t. angle of attack

In Fig. 20, the drag coefficient for the streamlined deck is the lowest among all the decks, while the drag coefficient for the bluff deck is the highest. Providing 100% fairing on the bluff deck is advantageous. The graph shows that 60% fairing falls within the range between the bluff deck and the fully faired deck. For a 0° angle of attack, the drag coefficient is the lowest, and it increases as the angle of attack increases for both negative and positive angles.

Fig. 23. Variation of Moment coefficient w.r.t. angle of attack

In Fig. 21, the moment coefficient for the streamlined deck is the lowest among all the decks, while the drag coefficient for the bluff deck is the highest. Providing 100% fairing on the bluff deck is advantageous. The moment coefficient is lower at a -5° angle of attack and increases as the angle of attack ranges from -4° to +5°.

4 Conclusion

As it was observed in the preceeding section, the fairings can influence the Static aerodynamic coefficients substantially towards stabilizing the system. In order to achieve greater economy 100 % fairing (fully faired) and 60% fairings (partial faired) were provided. Streamlined models performed better in comparison to unmodified decks and modified decks. Faired models show considerable change in the behaviour of the model towards improved stability as compared to the unmodified deck model. Partial faired model is less stable, though only marginally, compared to the fully faired model.

- Model 1 shows very less susceptibility to severe wind effects when tested at very high velocity (77m/s as per scaling) in comparison to Model 2 (which is the representation of Signature bridge deck model), Model 3 and Model 4 (fully and partial faired model).

- The aerodynamic characteristics of bridge decks significantly vary with their shape. Streamlined shapes showed superior performance in reducing drag and mitigating oscillations induced by wind forces compared to more blunt deck designs. Due to streamlined shape of deck in Model 1, the drag force and moment reduced approximately 60% of the one experienced by Model 2. As triangulated edges were introduced which scattered the wind flow in windward direction results in forming very less eddies in leeward direction.

- Model 3 (Full faired model) and Model 4 (partial faired model) reduced 35% and 13% of the wind effects experienced by Model 2.

- The study's findings have practical implications for bridge design, especially in regions susceptible to high wind events. Engineers can apply these insights to optimize deck shapes for new bridges or retrofit existing structures, thereby improving their wind resistance and extending their lifespan.

- For existing bridges, full length fairing is more beneficial than partial fairing as per the findings from this research work. The percentage of fairing can be adjusted according to economy and severity of the wind flow.

5 References

- Niels J. Gimsing, “Cable-Supported-Bridges-Concept-and-Design”.

- Holger. Svensson, Cable-stayed bridges : 40 years of experience worldwide. Ernst & Sohn, 2012.

- Javanmardi, K. Ghaedi, F. Huang, M. U. Hanif, and A. Tabrizikahou, “Application of Structural Control Systems for the Cables of Cable-Stayed Bridges: State-of-the-Art and State-of-the-Practice,” Archives of Computational Methods in Engineering, vol. 29, no. 3. Springer Science and Business Media B.V., pp. 1611–1641, May 01, 2022. doi: 10.1007/s11831-021-09632-4.

- Larsen and G. L. Larose, “Dynamic wind effects on suspension and cable-stayed bridges,” J Sound Vib, vol. 334, no. 1, pp. 2–28, 2015, doi: 10.1016/j.jsv.2014.06.009.

- M. Gu, Y. L. Xu, L. Z. Chen, and H. F. Xiang, “Fatigue life estimation of steel girder of Yangpu cable-stayed Bridge due to buffeting,” 1999.

- S. Laima, H. Feng, H. Li, Y. Jin, F. Han, and W. Xu, “A Buffeting-Net for buffeting response prediction of full-scale bridges,” Eng Struct, vol. 275, Jan. 2023, doi: 10.1016/j.engstruct.2022.115289.

- Masatsugu Nagai, Yozo Fujino, Hiroki Yamaguchi, and Eiji Iwasaki, Feasibility of a 1,400 m Span Steel Cable-Stayed Bridge, Journal of Bridge Engineering Volume 9, Issue 5, DOI: 10.1061/(ASCE)1084-0702(2004)9:5(444).

- Walter and Podolny, “20810498-16747637-Construction-and-Design-of-Cable-Stayed-Bridges-PODOLNY”.

- C. Qian, L. Zhu, Q. Zhu, Q. Ding, and L. Yan, “Pattern and mechanism of wind-induced static instability of super-long-span cable-stayed bridge under large deformation,” Journal of Wind Engineering and Industrial Aerodynamics, vol. 221, Feb. 2022, doi: 10.1016/j.jweia.2022.104910.

- Zengshun Chen, Hailin Huang, K.T. Tse, Yemeng Xu, and Cruz Y. Li, “Characteristics of unsteady aerodynamic forces on an aeroelastic prism: A comparative study,” 2020 Journal of Wind Engineering & Industrial Aerodynamics, Volume 205, 104325.

- E. Simiu, R. H. Scanlan. Wind Effects on Strctures. 2nd Edition, New York: Wieley, 1996.

- Sehoon Kim, Jung, Deok Keun Lee, “In-Situ Data-Driven Buffeting Response Analysis of a Cable-Stayed Bridge” 19(14), 3048; https://doi.org/10.3390/s19143048.

- Ming Li, Mingshui Li, and Yi Su, “Experimental determination of the two-dimensional aerodynamic admittance of typical bridge decks” Journal of Wind Engineering & Industrial Aerodynamics, volume 193, 2019, 103975.

- Baker, C., Hemida, H., Iwnicki, S., Xie, G., Ongaro, D., 2011. Integration of crosswind forces into train dynamic modelling. Proc. Inst. Mech. Eng. F J. Rail Rapid Transit 225 (2), 154–164. https://doi.org/10.1177/2041301710392476.

- M. Keerthana, P. Harikrishna, “Wind tunnel investigations on aerodynamics of a 2:1 rectangular section for various angles of wind incidence.” Wind and Structures, Vol. 25, No. 3 (2017) 301-328, doi.org/10.12989/was.2017.25.3.301

- Z. Ma, F. Xu, M. Wang, M. Zhang, and H. Zeng, “Design, fabrication, and dynamic testing of a large-scale outdoor aeroelastic model of a long-span cable-stayed bridge,” Eng Struct, vol. 256, Apr. 2022, doi: 10.1016/j.engstruct.2022.114012.

- Francesco Ricciardelli. (2002). “Pressure Distribution, Aerodynamic Forces and Dynamic Response of Box Sections.” J. of Wind Eng. & Ind. Aerodyn, Vol. 90(10), 1135-1150.

- M. Keerthana, K.P. Jaya, S. Selvi Rajan, Hephzibah Thampi, and R. Ravi Sankar, “Numerical Studies on Evaluation of Aerodynamic Force Coefficients of Cable-Stayed Bridge Deck.” Journal of wind and Engineering, Vol. 8, No. 2, July 2011, pp. 19-29