# Monolithic Design of Walls with Boundary Elements for Axial Force and Bending

| Rahul GUPTA M. Tech. (Structural Engineering) Shri G.S. Institute of Technology and Science Indore, MP rahul.1412.gupta@gmail.com Rahul Gupta, born 1998, received his civil engineering degree from the Indian Institute of Technology Roorkee, Uttarakhand, India. He is presently pursuing his M. Tech. degree programme at Shri G.S. Institute of Technology and Science, MP, India. His research focuses on analysis and design of structural wall systems for seismic forces. |  | Mahesh CHANDRA Assistant Professor Shri G.S. Institute of Technology and Science Indore, MP m.chandra9146@gmail.com Mahesh Chandra, born 1991, received his civil engineering degree from the Govt. Ujjain Engineering college, Ujjain, MP, India. He is completed his M. Tech. degree in Structural Engineering at SVNIT Surat, Gujarat, India. His research focuses on analysis and design of structural wall systems for seismic forces, estimation of seismic hazards losses and responses of overhead water tanks for dynamic action. |  | Rakesh Kumar KHARE Professor Shri G.S. Institute of Technology and Science Indore, MP rakeshkhare@hotmail.com Dr. Rakesh Kumar Khare, born 1964, has been working as a professor at Shri G.S. Institute of Technology and Science, MP, India from 2005. His research interests lie in finite element analysis, non-linear dynamic analysis, financial seismic risk analysis of buildings and structures, seismic hazard analysis and computer aided design of structures. |
| --- | --- | --- | --- | --- |

Abstract

Seismic ground motions produce strains which are represented by lateral forces for design of earthquake resistant structures. Reinforced Concrete (RC) walls in buildings are designed to resist these lateral forces. IS 13920: 2016 recommends boundary elements with dimension greater than the thickness of the wall to be provided at the ends of RC structural walls. This results in barbell-shaped wall sections. The prevalent method of design assumes boundary element to act as short axial column carrying axial load arising from gravity loads, and a force-couple idealised from the bending moment beyond the capacity of the wall web.  The presented study aims to design a monolithic barbell-shaped wall section for axial force and bending as per the limit state design method stated in IS 456: 2000. The expressions for moment of resistance proposed in this article presume a linear strain distribution profile for different conditions resulting in collapse of the walls. The wall sections with boundary elements are observed to have higher flexural capacity at the same axial load as per the suggested design method, compared to the design as per the present Indian Code of Practice for Ductile Design and Detailing of Reinforced Concrete Structures Subjected to Seismic Forces.

Keywords: RC Walls; Boundary Elements; Barbell-Shaped Wall Sections; Moment of Resistance; strain distribution.

1. Introduction

Structural Walls are built to increase the lateral stiffness of reinforced concrete (RC) buildings to resist lateral loading from wind and earthquakes. They may be provided as external walls, internal walls or form cores housing staircases, lifts and other utilities. RC walls are different from RC columns as IS 13920: 2016 [1] classifies them to be vertical members of RC buildings with a cross-sectional aspect ratio (larger dimension-to-smaller dimension of the cross-section) greater than 4. A ductile structural wall system (e.g., wall-beam-slab system) dissipates a much larger amount of energy than a moment-resisting frame system (i.e., column-beam-slab system) through inelastic actions of longitudinal and traverse reinforcements.

The present guidelines for the earthquake-resistant design of RC walls in India are based on rectangular wall sections that have been extensively tested across the globe to analyse their behaviour. Based on the height-to-width ratio, the primary reasons for failure in the wall vary. Squat walls, having a height-to-width ratio of less than 1, are subject to high nominal shear stress under lateral loads. Various experimental studies have observed inclined cracks indicating the formation of a diagonal compression strut system to resist lateral loads affecting the wall. This failure can be categorised as a diagonal tension failure due to inadequate horizontal and vertical reinforcement, or a diagonal compression failure due to concrete crushing. Slender walls with a height-to-width ratio greater than 2 are governed by their flexural strength. Past experiments and post-earthquake experiences document a predominant horizontal crack pattern in the lower hinge region of slender walls under in-plane lateral loading. Such walls fail in flexure due to the ductile yielding of vertical steel in flexural tension or crushing of concrete in compression.

Wall failure may also be observed due to a combination of failure in flexure and shear. Sliding shear failure at the wall foundation, construction joints, or critical sections joining the super-structure to the sub-structure and overturning failure due to the uplift of the foundation are observed in the absence of a design that provides proper anchorage to the walls.

Boundary elements are included at the ends of the wall to prevent buckling [2]. These elements may be columns forming a barbell-shaped cross-section with the wall or additional confined vertical reinforcement at the ends of the wall. They significantly improve the wall’s ability to sustain load reversals during an earthquake [2].

2. Design of RC walls for Axial Force and Bending as per Indian Standards

Medhekar and Jain 1993a [3] and 1993b [2] recommended the design of rectangular wall sections with expressions derived using principles of mechanics, strain compatibility conditions, provisions of limit state design as per Indian standards, stress-strain relation of concrete later adopted by IS 456: 2000 [4] and an idealised elastoplastic behaviour of steel reinforcement distributed uniformly in vertical and horizontal directions. The approach was adopted in IS 13920: 1993 [5].

For flexural tension failure, steel reinforcement at the extreme tension end yields before crushing of concrete at extreme compression end. With the following initial calculations, ϕ= 0.87 fy ρfck ; λ= PufcktwLw ; ρ= AsttwLw ; β= 0.87fyEs0.0035 ; where fck = characteristic cube strength of concrete, fy = yield stress for steel reinforcement, tw = thickness of wall web, Lw = total length of the wall, Ast = area of uniformly distributed vertical reinforcement, Es = elastic modulus of steel, and Pu = axial compressive force on the wall. The non-dimensional depth of neutral axis from extreme compressive fibre xu / Lw and the limiting value of the same xu*/Lw is calculated as follows

xuLw= ϕ+ λ2 ϕ+0.36 1 ; xu*Lw = 0.00350.0035+0.87fyEs  2 .

The moment of resistance of rectangular wall section for xu /Lw less than xu*/Lw is given by

MuRfcktwLw2= ϕ1+λϕ12-0.416xuLw- xuLw20.168+ β23 (3)

For flexural compression failure, steel reinforcement at the extreme tension end does not yield before crushing of concrete at extreme compression end. Here, xu /Lw is greater than xu*/Lw and is calculated by solving
α3 xuLw2+ α4 xuLw- α5=0 4

where, α3 = ϕ6β1xuLw-3 ; α4= ϕβ-λ ; α5= ϕ2β ; and moment of resistance of rectangular wall section, MuR, is given by

MuRfcktwLw2= α1 xuLw- α2 xuLw2- α3 - λ2  (5)

where, α1 = 0.36+ϕ1- β2- 12β ; α2= 1- β+ β23- 13β .

IS 13920: 2016 adopted the above equation with slight modifications. The ratio of yield strain for HYSD steel reinforcement to limiting compressive strain in concrete was modified, β= 0.002+0.87fyEs0.0035.

Subsequently,

xu*Lw = 0.00350.0035+(0.002+0.87fyEs)  6

The Indian Code suggests the design of boundary elements of a wall as an idealised force couple to resist the additional moment beyond the capacity of rectangular wall web along with the axial force in proportion with their area of cross-section.

The boundary elements are assumed to behave as short axial columns with dimensions greater than thickness of the wall web.

The minimum reinforcement requirements in RC walls are mentioned in Table 1 of IS 13920: 2016. The expressions have been adopted from Rohit et al. 2011 [6], considering the length of each boundary element to be at least 1.25 times the thickness of the wall web.

3. Extended Design Concepts

Dasgupta and Murty 2005a [7] and 2005b [8] elaborate the limit state of collapse for flexure defined in IS:456-2000 [4]. They observed a need of limiting strain for tension steel to avoid large curvatures in design of RC columns and walls as per limit state of collapse in compression. The paper proposed maximum tensile strain in steel reinforcement for limit state of collapse in compression to be twice the recommended tensile strain at failure by flexure, i.e., ϵst=2 ϵs=2 fy1.15 Es+0.002.

4. Moment of Resistance of RC wall with Boundary Elements

Fig. 1 Equivalent Section of RC Wall with Boundary Element

The moment of resistance of RC wall with boundary elements subjected to axial load and bending can be derived assuming a linear strain distribution across the barbell-shaped cross-section. The uniformly distributed vertical reinforcement in web can be represented as equivalent steel plate along the length of the web and extended till the ends of the wall representing a part of uniformly distributed vertical reinforcement in the boundary element. The remaining part of vertical reinforcement in the boundary element is also represented as an equivalent steel plate along the length of each boundary element. The equivalent section is shown below in Fig. 1.

The stress-strain curves assumed for the design are in accordance with Medhekar and Jain 1993a [3] as shown in Fig. 2. The failure of RC wall under uniaxial bending compression can be classified in three conditions depending on the strain distribution profile. The stress blocks, corresponding forces and their lever arm with respect to extreme tension fibre for the three cases are listed in Table 1.

Fig. 2 Representative Stress-strain Curves

(a) Concrete

(b) Steel

| Table 7.1: Forces and Corresponding Lever Arm for Wall Sections with Boundary Elements |  |
| --- | --- |
| Force | Lever Arm |
| Cc = 0.36 fck xu tw | Lw − 0.416 xu |
| Cbe = 0.446 fck tw Lw (tr – 1)lr | Lw(1 – lr/2 ) |
| C1 = (fy / γm) tw ρv xu (1 − β) | Lw − 0.5 xu(1 − β) |
| C2 = 0.5 (fy / γm) tw ρv βxu | Lw − xu(1 – (2/3) β) |
| T1 = 0.5 (fy / γm) tw ρv βxu | Lw − xu(1 + (2/3) β) |
| T2 = (fy / γm) tw ρv [Lw − xu(1 + β)] | 0.5[Lw − xu(1 + β)] |
| C3 = (fy / γm) tw ρv Lw(ρrtr − 1)lr | Lw(1 – lr/2 ) |
| T3 = (fy / γm) tw ρv Lw(ρrtr − 1)lr | Lw( lr/2 ) |
| Cs = (fy / γm) tw ρv (ρrtr − 1)lr [ xu Lw (1 + β) − (1 − lr)] | Lw [ lr − 0.5 { xu Lw (1 + β) − (1 − lr)}] |
| Pu | 0.5Lw |

The tensile stress less than the yield stress in extra steel reinforcement in the boundary element is neglected for ease in calculation. Force Cs is introduced on tension side to provide for this assumption.

The constants for the expressions stated below are:

ϕ= 0.87 fy ρfck ; λ= PufcktwLw ; ρbe= As,betwlbe ; β= 0.87fyEs0.0035 ;lr= lbeLw ;tr= tbetw ;

ρr= ρbeρv ;ρv=ρweb= As,webtwLw(1-2lr) ; xu,minLw= lr1-β ; xu,beLw= 1 - lr1 + β ; xu*Lw= 11 - β ;

Here, As,be = area of vertical steel reinforcement in each boundary element, lbe = length of each boundary element, tbe = thickness of boundary element, tw = thickness of wall web, and As,web = area of uniformly distributed vertical steel reinforcement in the wall web.

4.1. Case I : ( xu,min / Lw ) <  ( xu / Lw ) < ( xu,be / Lw )

When neutral axis lies inside the web and the tensile strain in the boundary element leads to yielding of the steel reinforcement in boundary element, the moment of resistance is calculated by the following expression.

MuRfcktwLw2= ϕ 1+ λϕ12-0.416 xuLw- xuLw2 0.168+ β23+ ϕ lr- lr2ρrtr-1+0.446 lrtr-10.416 xuLw- lr2          (7)

Where, the non-dimensional depth of neutral axis, xu / Lw , is given by  xuLw= ϕ+ λ-0.446 lrtr-12 ϕ+0.36   (8)

The case is further illustrated in Fig. 3, where (a) is the strain distribution profile for the case, (b) is the stress in the equivalent steel plate representing uniformly distributed vertical reinforcement in the web along the total length of the wall, (c) is the stress block for concrete along with the axial load on the wall, and (d) shows the stresses in extra steel reinforcement and concrete respectively in the boundary element.

Fig. 3 Case I : ( xu,min / Lw ) <  ( xu / Lw ) < ( xu,be / Lw )

4.2. Case II : ( xu,be / Lw ) <  ( xu / Lw ) < ( xu* / Lw )

When neutral axis lies inside the web and the tensile strain in the boundary element leads to only partial yielding of the steel reinforcement in boundary element, the moment of resistance is calculated by the following expression.

MuRfcktwLw2= ϕ 1+ λϕ12-0.416 xuLw- xuLw2 0.168+ β23+ ϕ lr- lr2ρrtr-1+0.446 lrtr-10.416 xuLw- lr2  - ϕ ρrtr-1xuLw 1+β-1-lr × 1-lr- 0.416 xuLw+ 12 xuLw 1+β-1-lr        (9)

Where, the non-dimensional depth of neutral axis, xu / Lw ,is given by

xuLw= ϕ+ λ-0.446 lrtr-1+ ϕ ρrtr-11-lr0.36+2 ϕ+ ϕ ρrtr-1(1+β)   (10)

Fig. 4 illustrates the case with parts (a) to (d) representing the same as in Fig. 3.

Fig. 4 Case II : ( xu,be / Lw ) <  ( xu / Lw ) < ( xu* / Lw )

4.2. Case III : ( xu* / Lw ) <  ( xu / Lw ) < 1

When no part of tensile steel reinforcement in the wall has yielded under combined action of axial load and bending the following expression shall be used to evaluate the moment of resistance of the wall section.

MuRfcktwLw2= α1 xuLw- α2 xuLw2- α3 - λ2     (11)

Where, the non-dimensional depth of neutral axis, xu / Lw ,is given by α3 xuLw2+ α4 xuLw- α5=0  12

Here, α1 = 0.36+ϕ1- β2- 12β ; α2= 1- β+ β23- 13β ;

α3 = ϕ6β1xuLw-3- lr- lr220.446 tr-1+ ϕ ρrtr-1 ;

α4= ϕβ-λ+ lr0.446 tr-1+ ϕ ρrtr-1 ; α5= ϕ2β .

Fig. 5 illustrates the case with parts (a) to (d) representing the same as in Fig. 3.

Fig. 5 Case III : ( xu* / Lw ) <  ( xu / Lw ) < 1

5. Boundary Element Conditions

The following conditions must hold true for calculation of moment of resistance of RC walls with boundary elements as per the above expressions –

- The length of each boundary element shall be greater than 1.25 times the thickness of the wall web, i.e. , lr ≥ 1.25 ( tw / Lw )

- The expression stated below for maximum length of boundary element must also hold true.

lr < ϕ732ϕ+0.36+ 0.446 tr-1     (13)

Fig. 6 P-M Interaction Curves with increase in thickness of boundary elements

Fig. 7 P-M Interaction Curves with increase in percentage of reinforcement of boundary elements

Fig. 8 P-M Interaction Curves with increase in length of boundary elements

Fig. 9 P-M Interaction Curves with increase in percentage of reinforcement in the wall web

6. Observations

The following observations can be inferred from the P-M Interaction curves shown above:

- An increase in the moment of resistance of wall sections with boundary elements is observed when thickness (Fig. 6), length (Fig. 7) and percentage of reinforcement (Fig. 8) in the boundary element increases.

- Moment of resistance of walls sharply declines with increase in axial load when no tensile steel yields in the wall.

- Moment of resistance of walls estimated by the suggested approach is much greater than obtained from the methodology suggested in IS 13920: 2016 at lower axial loads.

- At higher axial loads, the difference between the two approaches decreases.

- Displayed by Fig. 9, in the presence of high amount of vertical reinforcement in the wall web and greatly enlarged boundary element with large amount of vertical reinforcement, the flexural capacity of barbell-shaped wall sections calculated as per IS 13920: 2016 is higher than the suggested approach.

7. Results and Conclusion

The suggested approach observes conservativeness in the design of barbell-shaped RC wall section suggested by the Indian Code. The proposed method also observes an increase in the axial load capacity of the wall before failing with no tensile yielding in steel reinforcement.

At very high axial loads, the approach suggested by IS 13920: 2016 neglects the loss of tensile stress in the tension side boundary element leading to an unsafe design.

Hence the sections designed by the proposed approach provide a safer alternative than suggested by design as per IS 13920: 2016 while keeping in mind all the principles of mechanics, strain compatibilities and assumptions for limit state design.

The proposed method for designing barbell-shaped RC wall sections may also be more economical, as thinner sections with lesser reinforcement can be designed for lower axial loads while abiding by the boundaries of the theories of mechanics and assumptions of design adopted for a safe design in the Indian Code.

8. References

- IS 13920: 2016, Ductile design and detailing of reinforced concrete structures subjected to seismic forces - Code of Practice (first revision), BIS, New Delhi, India.

- Medhekar, M.S., and Jain, S.K., (1993b). Seismic behavior, design and detailing of RC shear walls, Part II: Design and detailing, Indian Concrete Journal, Vol. 67, 451-457.

- Medhekar, M.S. and Jain, S.K. (1993a). Seismic behavior, design and detailing of RC shear walls, Part I: Behavior and strength, Indian Concrete Journal, Vol. 67, 311-318.

- IS 456: 2000, Plain and Reinforced Concrete - Code of Practice (fourth revision), BIS, New Delhi, India.

- IS 13920: 1993, Ductile detailing of reinforced concrete structures subjected to seismic forces - Code of Practice, BIS, New Delhi, India.

- Rohit, D.H.H., Narahari, P., Sharma, R., Jaiswal, A. and Murty, C.V.R. (2011). When RC columns become RC structural walls, The Indian Concrete Journal, Vol. 85, No. 5, 35-45.

- Dasgupta, K. and Murty, C.V.R. (2005a) Seismic design of RC columns and wall sections: Part I – Consistent limit state design philosophy, The Indian Concrete Journal, March 2005, Vol. 79,  33-42.

- Dasgupta, K. and Murty, C.V.R. (2005b) Seismic design of RC columns and wall sections: Part II – Proposal for limiting strain in steel, The Indian Concrete Journal, March 2005, Vol. 79, 22-26.

- Majeed, A., Goswami, R. and Murty, C.V.R. (2015). Mechanics-driven Hand Calculation Approach for Obtaining Design P-M Interaction Curves of RC Sections. Indian Concrete Journal. Vol.89., No. 3, 59-68.

- Rajbanshi, S., Kumar, A. and Dasgupta, K. (2019). A comparative study of axial force—bending moment interaction curve for reinforced concrete slender shear wall with enlarged boundary element, In National Conference on Structural Engineering and Construction Management. Angamaly, Kerala, India Springer, Cham, 497-503.

- Kumar, A., Rajbanshi, S. and Dasgupta, K. (2019). Numerical Study on Lateral Deflection and Flexural Capacity of RC Shear Wall With and Without Enlarged Boundary Element, In National Conference on Structural Engineering and Construction Management. Springer, Cham, 489-496.

- Sharma, S., Gurjar, N., Menon, A., Srivastav, R. K., and Basu, D. (2022). Developing axial force bending moment interaction charts for the seismic design of RC slender shear walls with boundary elements, The Indian Concrete Journal, Vol. 96, No. 5, 29-58.

- Mondal, A. and Bhanja, S. (2023). Modifications proposed in IS: 13920 (2016) regarding shear wall design following the limit state design philosophy of national and international standards, Indian Concrete Journal. Vol. 97, 9-26.

- Mondal, K., Jaiswal, A. and Murty, C.V.R. (2014). Lateral load behaviour of squat RC structural walls, The Indian Concrete Journal, Vol. 88, No. 2, 37-47.

- Park, R., and Paulay, T. (1975), Reinforced Concrete Structures, John Wiley and Sons, Inc., New York, USA.

- Dasgupta, K. and Murty, C.V.R. (2014), Improved geometric design of earthquake-resistant RC slender structural walls, part I: parametric study, Journal of Engineering Mechanics, 140(5), 04014006.

- Dasgupta, K. and Murty, C.V.R. (2014), Improved geometric design of earthquake-resistant RC slender structural walls, part II: Design implications, Journal of Engineering Mechanics, 140(5), 04014007.

- IRC 112: 2020, Code of Practice for concrete road bridges (first revision), Indian Roads Congress, New Delhi, India.

- IS 1893: 2016 (Part 1), Criteria for earthquake resistant design of structures, Part 1 general provisions and buildings (sixth revision), BIS, New Delhi, India.

- Mondal, A. and Bhanja, S. (2023), Shortcomings of shear wall design as per is:13920–2016, Materials Today: Proceedings, 93, 402-407.