Robust unsupervised multi-scale structural damage detection from vibration video

Subhajit Banerjee1 and T. Jothi Saravanan1

1 School of Infrastructure, Indian Institute of Technology Bhubaneswar, Odisha, India
tjs@iitbbs.ac.in

Abstract. Structural Health Monitoring (SHM) is pivotal in identifying and mitigating damage risks in large structures, thereby ensuring their safety and longevity. In the realm of structural vibration-based damage identification, Computer vision (CV)-aided SHM systems offer significant advantages over conventional sensor systems, including the capability to monitor extensive and inaccessible areas with lower installation costs. The present work introduces a novel approach for non-contact CV-based structural vibration response measurement, damage localization and severity quantification from vibration video data. The videos are pre-processed using homography transformation in situations where the image and the structural planes are skewed with respect to each other. The Features from Accelerated Segment Test (FAST) algorithm, with an adaptive threshold strategy, is employed to identify high-quality features within the Region of Interest (ROI) surrounding each sensor location. Subsequently, these feature points are tracked across each video frame using the Kanade-Lucas-Tomasi (KLT) algorithm, to obtain the vibration response in physical units. A hybrid combination of unsupervised learning model and a stochastic subspace state-space system identification algorithm, N4SID is used to extract the state-space matrices from the displacement response. Further Stochastic damage locating vector (SDLV) approach, aided by an improved Bayesian inference-based stochastic model updating (BI-SMU) algorithm, is employed to precisely identify the damaged elements of the structure. The proposed approach is validated on a numerical truss model with simulated damage scenarios and then implemented on the damage localization for a photo-realistic synthetic truss bridge model from its vibration videos. The method shows excellent accuracy in effectively locating structural damages across varying severity levels, thereby enhancing its potential in practical SHM applications.

Keywords: Homography transform, Features from Accelerated Segment Test, Bayesian inference-based stochastic model updating

- Introduction

Civil engineering constructions face diverse pressures over their lifetime, with unexpected forces such as intense winds and seismic activities posing the risk of considerable harm or even disastrous collapse. To reduce these potential risks and bolster structural security and durability, Structural Health Monitoring (SHM) systems play a crucial role by providing continuous information on the condition of structures, allowing for early identification of problems and prompt maintenance. Technological progress in design and building has greatly enhanced SHM techniques, making damage evaluation more effective and precise. Traditional vibration-based structural damage identification methods, which relied on physical sensors, used vibration characteristics like mode shapes, modal frequencies, and acceleration responses due to their sensitivity to damage. These methods were effective in detecting damage through shifts in natural frequencies and mode shapes [1], but had certain limitations, such as mass-loading errors from sensor installation and constraints imposed by site conditions [2]. Researchers also utilized techniques involving contactless sensors, such as Laser Doppler Vibrometer (LDV) and Microwave Interferometer (MI) [3], which accurately measure vibration responses without physically interacting with the structure. Saravanan and Rao [4] developed an explicit damage visualization technique for PZT-excited aluminium plates, using LDV to record out-of-plane flexure responses. However, these methods also presented challenges, including high costs and labour-intensive setups.

As a result, the field has seen a growing interest in non-contact computer vision (CV)-based vibration measurement techniques. These methods utilize video cameras as low-cost, portable sensors and apply advanced image processing techniques to effectively detect structural damage. Wadhwa and Freeman [5] introduced the phase-based video motion magnification (PBVMM) technique for enhancing the visibility of subtle structural vibrations, optimized for low-cost hardware and video post-processing. Chen et al. [6] proposed a novel technique by combining PBVMM with point tracking for displacement extraction, and demonstrated its effectiveness on lab-scale models, compared to LDV measurements. Sarrafi et al. [7] utilized phase-based motion estimation (PBME) with digital video cameras to identify damage in wind turbine blades. Saravanan and Nishio [8] further validated PBVMM for extracting structural displacement from video footage of a highway viaduct light pole, with accuracy comparable to LDV, especially in low-amplitude conditions. Saravanan et al. [9] employed a two-dimensional digital image correlation (2D-DIC) technique for strain field data acquisition to monitor local damage which showed results consistent with strain gauge measurements. Some researchers [10, 11] have also proposed using synthetic data to complement real-world images, where automatic annotation with class-specific masks eliminates the need for time-consuming manual labelling.

The growing complexity of structures has driven the adoption of improved real-time damage monitoring systems which are more robust in dynamic and uncertain environments, and have better sensitivity to minor changes in structural properties. Researchers have increasingly adopted time-series-based methods for structural damage localization by analyzing dynamic responses over time. These techniques effectively detect and localize damage, offering insights into the formation and growth of structural issues. Sohn and Farrar [12] introduced a novel two-stage approach using time series analysis, by merging auto-regressive (AR) and auto-regressive with exogenous inputs (ARX) techniques, to detect damage locations in vibrating structures. Bhowmick et al. [13] introduced a novel real-time damage identification methodology involving recursive canonical correlation analysis to continuously update modal parameters as structural conditions change and demonstrates its effectiveness through comprehensive experimental validation.

In the past decade, there has been a significant increase in the use of stochastic-based damage localization techniques in structural damage detection, highlighting their importance in managing the complexities and uncertainties of real-world structural assessments. Bernal [14] developed an algorithm to detect damage by constructing minimal state-space realizations and extracting flexibility matrices from sensor data. Bernal [15] further developed a structure-independent technique to localize damage in linear time-invariant structures by computing damage locating vectors (DLVs) from changes in measured flexibility, which identify damaged areas by inducing zero magnitude stress fields. Expanding on this, Bernal and Gunes [16] proposed a flexibility-based damage characterization method that analyzes changes in a flexibility-proportional matrix under stochastic excitation, allowing damage location and quantification without relying on a structural model. Bernal [17] further tackled the challenge of localizing structural damage by analyzing changes in flexibility due to operational or ambient vibrations, introducing a method to extract stochastic DLVs (SDLVs) from the null space of the flexibility change matrix, even when the spatial distribution of the generating input is unknown. The key contribution of the study involves deriving a connection between the state-space matrices Ac,Cc associated with a collocated input distribution and the observed output. The approach is grounded in the assumption that damage does not affect inertial properties, and it allows for effective damage localization without the need for complete input knowledge. Tan et al. [18] developed a Bayesian inference (BI)-based objective function for damage identification in space truss structures, using the L-SHADE algorithm to minimize the function and assess damage severity. Ni et al. [19] introduced a CV-based, non-destructive SHM approach for truss bridges, combining machine learning algorithms (FairMOT and Harris corner matching) with stochastic subspace identification and Bayesian inference for multi-level damage localization.

Deep learning models like Recurrent Neural Networks (RNN) and Long Short-Term Memory (LSTM) networks have also been used for dynamic inferential modelling. Li et al. [20] applied LSTM for state space realization in two industrial case studies, comparing it with linear state space modeling and statistical methods. Results showed that LSTM sometimes underperformed but also demonstrated superior performance in other cases compared to linear methods.

The present study aims to present a novel non-contact structural damage localization technique that integrates computer vision with stochastic damage localization. To enhance the precision of structural damage evaluation, a BI-based method is incorporated to estimate the severity of detected damage. The proposed approach is initially tested using vibration responses from a simulated numerical truss with artificial damage. The effectiveness of this CV-based localization method is further validated using vibration videos from a photo-realistic synthetic truss bridge model, with its geometry and deformation derived from finite element analysis and featuring various damage scenarios.

- Methodology

Structural damage localization and severity quantification, in the present study, has been achieved through three major phases, as illustrated in Fig. 1: (1) Displacement extraction from vibration video data using adaptive FAST feature detection and KLT tracking to obtain the structural response in terms of nodal displacements; (2) System identification through the numerical algorithms for subspace state space system identification (N4SID) to estimate state-space matrices and calculate SDLVs, which localize potentially damaged (PD) elements by inducing zero stress in damaged areas under unknown excitation; and (3) Bayesian inference-based model updating (BI-SMU) approach to eliminate false positives (FPs) from SDLV results by comparing the residual stiffness in the PD elements, obtained after convergence of the Markov Chain. A detailed explanation of the algorithms is discussed in the subsequent subsections.

Fig. 1. Flowchart of the proposed methodology

2.1    Vibration displacement extraction

The non-contact computer vision-based approach presented in the present work for vibration displacement measurement is characterized by a sequence of critical phases, encompassing: (i) Geometric transformation correction (for cases in which the image plane is not parallel to the truss front plane, (ii) Critical feature detection using FAST algorithm, followed by (iii) feature tracking and displacement extraction using KLT algorithm, to fetch displacement response. In this first step, distinctive features such as the edges and corners of the target object, are identified within a specified ROI. Changes in space and time are then scrutinized within this ROI area. This process of examining spatial and temporal changes extends across complete sequences of images or frames to achieve object tracking.

2.1.1 Geometric transformation correction

While dealing with situations where the structural plane deviates from being parallel to the image plane, specific additional pre-processing becomes necessary, since the scale factor cannot be treated as a constant due to the skewed nature of the image plane. To address this challenge, the homography rectification technique, as proposed by Loop and Zhang [21] is employed on the object plane. Consider two distinct image planes denoted as θ and θ', any given object points s and s' present in corresponding images (as illustrated in Fig. 2) are linked through a mathematical transformation called homography (denoted as H) as in Eq. 1:

s'=Hs                                                               (1)

Considering both the reference and skewed images frames to be two-dimensional, the homography transformation matrix H is a 3×3 matrix, with entries derived from a set of corresponding point pairs {s,s'}.

Fig. 2. Corresponding points mapping between non-parallel image planes θ and θ'

2.1.2 Critical feature detection

The process of capturing temporal displacement responses from the input vibration video is initiated with the initial frame through the detection of critical features of the target object in motion, within a particular ROI. The identification of these features is facilitated by feature detection algorithms. In this regard, the FAST algorithm [22] has been employed in this study because of its enhanced accuracy in detecting critical features within ROI, in comparison to the conventional Harris Corner detector [23], while also ensuring efficient computational speed. Corner point selection is performed by analyzing circular pixel intensity variation by a threshold ‘t’. To retain the robustness of the algorithm in dealing with different images obtained under different ambient conditions (brightness, noise level), an adaptive local threshold value (t0) is considered in the present work, as in Eq. 2.

to=m⋅j=1qImax-j=1qIminqI                                           (2)

where Imax, Imin and I represent the maximum, minimum, and average value of greyscale in the ROI, q is the number of pixels in the ROI and m is a proportion ratio, generally considered as 3 [22].

2.1.3 Critical feature tracking and displacement extraction

Considering the vibration motion of the structure used in the study is relatively limited, an optical flow-based tracking method, the KLT algorithm has been adopted in this study. The KLT tracking algorithm is a popular computer vision technique used for tracking and estimating the displacement of features (corner points) in consecutive frames of an image sequence (video of an object in motion). It originates from a synthesis of works by Lucas et al. [23], Tomasi and Detection [24], and Tomasi and Shi [25].

The target object within ROI is tracked and its displacement across the frames is analyzed in two primary steps using the KLT algorithm: (1) locate a target within the initial frame and capture its distinctive features; these identical features are extracted from every subsequent frame, (2) measure the offset between a feature from the first frame to the rest of the frames. The steps are repeated until all features within the ROI are successfully tracked. The KLT algorithm is applied to the structural vibration video to extract the displacement response of the structure in pixels. The extracted displacement is converted from pixel to meter by multiplying with a suitable scale factor.

  - Structural damage identification

Damage detection process is performed on the displacement response data (X) obtained from the previous section. Generally, for a linear time-invariant system when the input is stochastic (or, unknown), the state-space model can be expressed as:

xk+1=Acxk+wk                                                    (3)

yk=Ccxk+vk                                                      (4)

where xk∈RN×1 is the state vector at time k, yk∈Rn×1 is the output (response) vector at time k; for the stochastic input case, wk and vk = (assumed to be) independent stationary white noise sequences. The matrices Ac∈RN×N, Cc∈Rn×N are the system matrix, and the state-to-output mapping matrix respectively.

Since in the present work, the input excitation to the structure is unknown and random (stochastic), a stochastic subspace identification (SSI) approach is employed to reconstruct the discrete state-space matrices (Ac and Cc), which will be further required in damage localization. Then, the SDLV algorithm is adopted to roughly locate the damage location(s) from the already estimated Ac and Cc matrices.

2.2.1 Estimation of the system matrices using N4SID

N4SID belongs to the group of stochastic identification methods that aim at approximating the state sequence of the linear time-invariant system and use the approximated state in a second step to estimate the system matrices. The N4SID method necessitates either acceleration or displacement responses alongside the optimal system order. This order corresponds to the arrangement of singular values within the Hankel matrix. Unlike earlier literatures which used stabilization charts for deriving state-space model order, the present study employs an innovative approach by blindly extracting model order using unsupervised learning, Principal component analysis (PCA). PCA effectively gauges the number of significant principal components within the vibration signal, thereby approximating the number of substantial modes.

First, the mean subtracted displacement matrix Xm is calculated from X. Singular value decomposition (SVD) is applied to Xm to obtain eigenvalues of its covariance matrix, using Eq. 5.

Xm=UΣVT                                                        (5)

where, U and V are right and left singular matrices of Xm, and Σ is a diagonal matrix representing the eigenvalues of Xm. The eigenvalues are sorted and the optimal order for N4SID is identified by the eigenvalue index that meets this criterion: the eigenvalue must be greater than 50% of the preceding value and less than 10% of the initial eigenvalue. Further, the N4SID algorithm, with the order of the system identified, is employed to compute the state-space matrices Ac and Cc from the displacement or acceleration responses. The ‘field-measured’ modal frequencies of the structural system can also be extracted from the state matrix Ac by computing its eigenvalues.

2.2.2 Detection of potential damaged elements using SDLV approach

The SDLV technique extends the Damage Locating Vector (DLV) technique proposed by Bernal and Gunes [16], which relies on null space vectors of the change in flexibility proportional matrix (∆ZT) between damaged and undamaged states to yield zero stress fields over damaged elements, under unknown loading. The flexibility proportional matrix Z and its change (∆Z) is expressed as:

Z=-CcAc-(p+1)CcAc1-pCcAc-p*I0                                     (6)

∆Z=Zd-Zu                                                     (7)

where, A∈RN×N and C∈Rm×N with m and N being the number of inputs and number of components of state vector respectively; variable p varies according to the measurement signal, which is taken as 0, 1 and 2 for displacement, velocity, and acceleration measurements respectively; the symbol * applied to the matrix signifies the pseudo inverse. The subscripts ‘d’ and ‘u’ in Eq. 7 corresponds to the damaged and the undamaged structures.

The SVD of ∆ZT is performed, and the SDLVs V0 are obtained as the null space of ∆ZT:

(ΔZ)TV0≈0                                                           (8)

The application of load vector V0 to the undamaged structure should lead to zero-stress field among the damaged elements. The optimal number of significant SDLVs (qn) can be calculated using Eq. 9 (Bernal (2006)).

qn=0.5×number of ξ≤0.1|ξ=SiSi,max                              (9)

where, Si’s are the singular values of (ΔZ)T. A finite element (FE) model of the structure is created, and the SDLV vectors are applied as the force vectors on the FE model at the sensor locations to estimate the member stresses (σi's). Further the Weighted Stress Index (WSI) is determined for each of the members in the structure, facilitating the identification of PD elements. The computation of WSI is illustrated by Eq. 10:

WSI=i=1qn|σi||σi|max≤0.1                                                  (10)

PD elements are detected from the WSI plot for the damaged case using an appropriate WSI tolerance value.

  - Precise damage localization

The application of SDLV approach may sometimes result in identifying elements as damaged when they are actually not, leading to FP cases. To address this, the present study employs a BI-SMU approach [26] to remove FP cases from the PD elements obtained from the previous section, thereby ensuring precise damage localization.

2.3.1 Gaussian process regression

Structural damage, which can arise from changes like reduced stiffness (due to cracking, corrosion, or spalling) or mass alterations (also from corrosion or spalling), influences the structure's modal frequencies. The relationship between the relevant damage parameter of the PD elements and the decrease in structural modal frequencies corresponding to various combinations of this parameter in the PD elements, is established through Gaussian process metamodel (GPM) [26].

GPM is a robust Bayesian machine learning method that models intricate non-linear connections between input variables (X) and their corresponding output responses (y), while also accounting for uncertainty in the predictions. A GPM with zero mean and squared exponential kernel is used in the present study for the best fit, and can be expressed as:

uX ~ GP(0,kX,X')                                            (11)

The squared exponential kernel function can be described as in Eq. 12 as:

kX,X'=exp⁡(-12X-X'TΣ-1X-X')                       (12)

where Σ denotes hyperparameters controlling the smoothness of the function. The optimization of the hyperparameters Σ has been performed using the Limited-Memory Broyden-Fletcher-Goldfarb-Shanno (L-BFGS) optimizer.

2.3.2 Bayesian Inference-based stochastic model updating

The basic motive of BI is to refine prior beliefs about a parameter or model using observed data, resulting in a posterior probability distribution. It uses Bayesian statistics to handle uncertainties in both the model and measurements, offering a probabilistic method for model improvement. Consider a model that assumes the data y follows a certain probability distribution, which depends on an unknown parameter θ. Assuming a prior distribution about θ (p(θ)), the parameter θ can be updated based on observing the data y, as follows:

pθy=pyθp(θ)p(y)                                                 (13)

where p(θ|y) is the posterior distribution, representing updated beliefs about the parameters after observing the data, p(y|θ) denotes the likelihood, quantifying the probability of observing the data given the model parameters and p(y) is the marginal, serving as a normalizing constant, which can be further expressed as:

py=θpyθp(θ)                                             (14)

However, the exact computation of the above integral becomes increasingly difficult to deal with in higher dimensions. Hence, the estimation of the posterior distribution requires approximate computation of the above integral, which is mostly performed using Markov chain Monte Carlo (MCMC) simulations. MCMC estimates the posterior distribution of parameters by generating samples from a target distribution via a Markov chain, exploring the parameter space and converging to the posterior through iterative acceptance or rejection of proposed values.

The ‘field-measured’ modal frequency reductions obtained from video data (from Section 2.2) are combined with the GPM predictions to construct the ‘likelihood’ function. The posterior probability distributions of the updated damage parameters computed using the MCMC simulations will help estimate the damage parameter of the structural elements. The precise damaged element can be detected based on the level of change in damage parameter of the elements, i.e. the damaged element/s will show a significantly high change in damage parameter over several iterations, compared to the FP elements.

- Validation of the proposed methodology

  - Numerical truss structure

The proposed methodology has been first validated on the FE model of a numerical truss model, created on MATLAB interface. The truss has a total length of 3m and a height of 1m. The entire geometry of the numerical truss model is illustrated in Fig. 3. The material used for the truss member has an Young’s modulus as 210 GPa, and a density of 8000 kg/m3.

- (b)

Fig. 3: Numerical truss model with (a) member indices and (b) member geometry

To enhance accuracy of FE analysis, a condensed system mass matrix is created instead of the truss's lumped mass matrix. Eq. 15 delineates the condensed mass matrix [Me] and stiffness matrix [Ke] of a truss element in its local coordinates.

Me=mE62002100120021001      ;    Ke=AEL1   00   0-10   00-1000   10   00           (15)

where, m is the mass per unit length, E is the Young’s modulus of member material, A is the cross-sectional area of the member, and L is the length of the truss member. Due to a 10 kN impulse applied on the top left truss joint, displacement data is gathered from all truss joints, excluding the two support joints using Newmark beta algorithm with α = 0.25 and β = 0.5. Real-world condition is emulated through introduction of Gaussian white noise of intensity 30 dB to the simulated signals. To validate the damage localization performance, three damage scenarios are generated in the numerical truss. These scenarios involve reducing the cross-sectional area of the truss member by 50%, thereby diminishing its flexibility by the same proportion. The three instances of damage differ in terms of the location where the reduction in cross-sectional area occurs. Damage case-I represents damage in member 1, damage case-II represents damage in member 7 and damage case-III represents damage in member 9.

  - Damage identification

PCA is performed on the noisy four-channel displacement response obtained from the numerical model. Fig. 4 illustrates the eigenvalue decomposition (EVD) plot of the covariance of the displacement matrix. With an optimal system order of four, The N4SID function is then employed to extract the subspace system matrices Ac and Cc based on the displacement data. The actual modal frequencies of the numerical truss for all damage cases as well as the undamaged case, computed from the EVD of the state matrix Ac.

Fig. 4: EVD plot of covariance of displacement matrix

The change in flexibility proportional matrix (ΔZ) is computed using the system matrices as per Eqs. 6 and 7. The SDLVs are extracted as the null space vectors of (ΔZ)T matrix. The significant SDLVs among them (computed from Eq. 9) are applied as force vectors on the FE model of the undamaged truss at the joints from where displacements are extracted. The member stresses obtained for all SDLVs (for each damage case) are recorded and WSI is calculated for all the truss members using Eq. 10. The suspected PD elements are identified using a WSI tolerance level of 0.1. Table 1 depicts the damage identification results obtained after the SDLV step. The WSI plots for all damage scenarios are shown in Fig. 5. The results indicate that the technique effectively identifies the correct damaged elements in all damage cases. However, for some damage cases, it also yielded false positive results additionally. This can be due to support conditions and topology of the structure.

Table 1: Results of damage identification in numerical truss using SDLV approach only

| Damage case | Actual damage | Identified damage | No. of FP cases |
| --- | --- | --- | --- |
| I | 1 | 1, 4 | 1 |
| II | 7 | 6, 7 | 1 |
| III | 9 | 9 | 0 |

- Damage case-I

- Damage case-II

- Damage case-III

Fig. 5: WSI plots for all damage cases as estimated by the SDLV approach (black arrows represent actual damage detected, red arrows represent FP cases)

  - Precise damage localization

GPM is used to establish the relationship between the remaining stiffness of the suspected PD elements and the corresponding reductions in modal frequencies resulting from stiffness degradation. For example, in case of damage case-I, the residual stiffness of the suspected PD elements 1 and 4 are used as the GPM inputs. As the elemental stiffness varies in FE analysis, the modal frequencies change accordingly. The reductions in modal frequencies for first four vibration modes, corresponding to the residual stiffness of the truss elements, are considered as outputs for the metamodel.

Next the BI-SMU method is employed for precise damage localization and severity quantification. The log-likelihood function is formulated by combining the actual frequency reductions with GPM predictions, through the estimation of Maximum aposteriori probability (MAP). The residual stiffness of the suspected PD elements is believed to be random variables having a Uniform prior distribution between 0 and 1. The posterior probability distributions of the residual stiffness are then simulated using the MCMC algorithm over one lakh iterations. The initial assumption of the residual stiffness for all PD elements is taken as 1 or 100%. At each iteration step, the belief about the residual stiffness of the PD elements is updated with the current assumption of residual stiffness, if the corresponding log-likelihood exceeds that corresponding to the existing belief of residual stiffness by some tolerance (in this case taken as a random value between 0 and -0.00001), otherwise is updated with the previous assumption in the Markov chain. The precise damage localization outcomes for the damage cases I and II (having FP cases) are illustrated in Fig. 6. For example, in case of damage case I (as shown in Figure 6(a)), the stiffness in truss element 1 reduces to around 50% of its initial value after thousand iterations, while that in truss element 4 remains stable, in close proximity of its initial value. Thus, the proposed methodology could successfully locate the damaged elements correctly for all damage cases of the numerical truss.

(a)

(b)

Fig. 6: Precise damage localization results of (a) Dam-I, and (b) Dam-II

Table 2: Precise damage localization of all three cases of numerical truss

| Damage case | Actual damage | Identified damage |
| --- | --- | --- |
| I | 1 | 1 |
| II | 7 | 7 |
| III | 9 | 9 |

  - Experimental truss bridge model vibration video

The proposed approach is applied to the vibration video data of a photo-realistic synthetic model (from Narazaki et. al. [10]), consisting of 14 bays with dimensions of 5.518m in length, 0.4m in height, and 0.3937m in width. A total of four videos are analyzed: one undamaged and three with varying damage scenarios (damage cases are DC-I, DC-II and DC-III). Out of the three videos with damage incorporated, that with DC-I has the image plane parallel to the structural plane (as depicted by category “Parallel” in Table 3), while the other two have the image plane skewed with respect to the structural plane (as depicted by category “Unparallel” in Table 3), indicating measurement noise. Each video captures the vibration of only the first seven bays of the truss, as all the damaged elements are located within these bays. Fig. 7 shows the portion of the truss in the video frame, with all member indices marked.

Table 3: Ground truth features of the damage cases used for analysis

| Damage case | Category | Damaged element no. | Residual stiffness (%) |
| --- | --- | --- | --- |
| I | Parallel | 18 | 47.3 |
| II | Unparallel | 6 | 42.5 |
| III | Unparallel | 16 | 27.5 |

Fig. 7: Illustration of truss in video frame (Truss joint ID (yellow) and member ID (red))

  - Geometric transformation correction

Fig. 8(a) and 8(b) depicts the image frames for “Parallel” case and “Unparallel” case respectively. The skewness of the image plane with respect to the structural plane in “Unparallel” cases, will result in erroneous displacement extraction. In the present study, homography rectification is employed to identify a transform H to map the image points on the target frame (“Unparallel” case) to the corresponding points in the reference frame (“Parallel” case), with the constraint that the point correspondences are co-planar. The estimated transform H can be further performed on the skewed frame to convert it into the ‘Parallel’ type image (as shown in Fig 8(c)).

  - Displacement extraction from vibration video

The process of displacement extraction initiates with selecting ROI around the target object, here, each joint of the truss (as shown in Fig. 9(a)). Once the ROI is selected, corners in the ROI are identified using the adaptive FAST technique (as shown in Fig. 9(b)). KLT feature tracking algorithm is used to extract the displacement of centroid of each joint along the video frames. The spatio-temporal displacement values are converted from pixels to ground coordinates. A comparison between the extracted displacement (in m) and the response obtained from sensors (in m) reveals a good correlation. Fig. 10 shows the vertical displacement response of truss node 1 (of undamaged truss) compared with the corresponding ground truth, with a RMSE value of 0.089 mm, suggesting strong concordance.

(a)

(b)

(c)

Fig. 8: Vibration video frames: (a) “Parallel” case, (b) “Unparallel” case and (c) Recovered frame corresponding to (b)

- (b)

Fig. 9: Illustration of feature detection using FAST: (a) Selected ROI, (b) Identified corners in ROI

  - Damage identification

Vibration videos corresponding to three damage scenarios of the truss are analyzed to validate the proposed method. Principal Component Analysis (PCA) is performed on the displacement data for both the undamaged and damaged truss, with the optimal system order determined to be 5, as depicted in Fig. 11. The system matrices, as well as the actual modal frequencies corresponding to each scenario, are extracted using N4SID.

Fig. 10: Vertical displacement response (in m) for video estimation and ground truth

Fig. 11: EVD plot of the covariance of the displacement matrix of the experimental truss

For each damage case, SDLVs are computed from the SVD of (ΔZ)T, and the significant ones among them are applied at sensor locations on the FE model of the undamaged truss. The member stresses and consequently, the WSI for each truss member are calculated. The suspected PD elements are identified using a suitable WSI tolerance index, and are tabulated in Table 3, along with the FP cases generated. The damage detection results when compared to that obtained from existing CV-based damage detection approach [19] on the same structural vibration video, ensures significantly fewer FP cases generated (also shown in Table 4). The WSI plot for the three damage scenarios are illustrated in Fig. 12.

Table 4: Results of damage identification for vibrating truss video using SDLV approach only

| Damage case | Actual damage | Identified damage | No. of FP cases | No. of FP cases in [19] |
| --- | --- | --- | --- | --- |
| I | 18 | 18 | 0 | 4 |
| II | 6 | 6, 7 | 1 | 5 |
| III | 16 | 16 | 0 | 5 |

- Damage case-I

- Damage case-II

- Damage case-III

Fig. 12: WSI plots for the five damage scenarios obtained using the SDLV technique

  - Precise damage localization

The proposed BI-SMU technique is implemented on damage case-II in order to identify and remove the FP case. A GPM is established between the residual stiffness (damage parameter) of the suspected PD elements, and the corresponding reductions in modal frequencies for first five vibration modes. The log-likelihood function is formulated by combining the actual frequency reductions, with GPM predictions, through the estimation of MAP. The posterior probability distributions of the residual stiffness are then simulated using the MCMC algorithm over one lakh iterations. The precise damage localization outcome for the damage case II is tabulated in Table 5 as well as illustrated in Fig. 13. The stiffness in truss element 6 reduces to around 50% of its initial value after two thousand iterations, while that in truss element 7 remains stable, in close proximity of its initial value, indicating FP element. Thus the proposed methodology showed excellent performance in accurately detecting and localizing damage in truss structure from its vibration video only.

Table 5: Damage localization results of all three damage cases for the experimental truss

| Damage case | Category | Actual damage | Identified damage |
| --- | --- | --- | --- |
| I | Parallel | 18 | 18 |
| II | Unparallel | 6 | 6 |
| III | Unparallel | 16 | 16 |

Fig. 13: Precise damage localization result of Damage case-II of experimental truss

- Conclusion

The present work is an attempt towards the growing necessity of introducing state-of-the-art technologies in modern civil engineering construction and maintenance so as to detect potential risks early, thereby increasing durability. The study presents an innovative vibration-based structural damage detection approach from vibration video through the integration of computer vision, stochastic subspace analysis and Bayesian inference. Homography transform is utilized to pre-process the videos with skewed image frames. An optical flow tracking technique, KLT tracks the critical features identified using adaptive FAST, along the video frames to yield the displacement response. SDLV approach is employed to identify the PD elements in each damage case. However, in some cases, SDLVs yield FP elements additionally, which are further removed by using the BI-SMU technique. Thus the methodology is successfully validated on a numerical structural model as well as on the vibration video of a photo-realistic synthetic structural model with varying damages. Though the proposed algorithm has been designed for truss-like structures, its applicability can also be extended to framed structures can be a scope for further research. The robustness and accuracy of the proposed methodology makes it suitable for real-world SHM applications.

Acknowledgment

The corresponding author of this article gratefully acknowledges support from the Indian Institute of Technology Bhubaneswar under grant SP144. Also, the authors acknowledge the financial assistance from fellowship to the first author by the Ministry of Education, Government of India.

References

- Fan, W. and Qiao, P. (2011): Vibration-based damage identification methods: a review and comparative study. Structural health monitoring, 10(1), 83–111.

- Ashory, M. (1998): Correction of mass-loading effects of transducers and suspension effects in modal testing. In: Proceedings of the 16th International Modal Analysis Conference, 3243.

- Farrar, C. R., Darling, T. W., Migliori, A., and Baker, W. E. (1999): Microwave interferometers for non-contact vibration measurements on large structures. Mechanical Systems and Signal Processing, 13(2), 241–253.

- Saravanan, T. J., Gopalakrishnan, N., and Rao, N. P. (2015). Damage detection in structural elements through propagating waves using radially weighted and factored RMS. Measurement, 73, 520–538.

- Wadhwa, N., Rubinstein, M., Durand, F., and Freeman, W. T. (2013): Phase-based video motion processing. ACM Transactions on Graphics (TOG), 32(4), 1–10.

- Chen, J. G., Wadhwa, N., Cha, Y.J., Durand, F., Freeman, W. T., and Bu ̈yu ̈ko ̈ztu ̈rk, O. (2015): Modal identification of simple structures with high-speed video using motion magnification. Journal of Sound and Vibration, 345, 58–71.

- Sarrafi, A., Mao, Z., Niezrecki, C., and Poozesh, P. (2018). Vibration-based damage detection in wind turbine blades using phase-based motion estimation and motion magnification. Journal of Sound and Vibration, 421:300–318.

- Saravanan, T. J., Gopalakrishnan, N., and Hari, B. K. (2019). Damage identification in structural elements through curvature mode shapes and nonlinear energy operator. Compos. Mater. Engg., 1(1), 33–48.

- Saravanan, T. J., Nishio, M., and Suzuki, T. (2019). Computer vision-based in-plane strain field monitoring for identification of local damages in structural members. Structural Health Monitoring.

- Zhai, G., Narazaki, Y., Wang, S., Shajihan, S. A. V., & Spencer Jr, B. F. (2022). Synthetic data augmentation for pixel-wise steel fatigue crack identification using fully convolutional networks. Smart Struct Syst, 29(1), 237-250.

- Yao, Z., Jiang, S., Wang, S., Wang, J., Liu, H., Narazaki, Y., ... & Spencer Jr, B. F. Intelligent crack identification method for high‐rise buildings aided by synthetic environments. The Structural Design of Tall and Special Buildings, e2117.

- Sohn, H. and Farrar, C. R. (2001): Damage diagnosis using time series analysis of vibration signals. Smart materials and structures, 10(3), 446.

- Bhowmick, B., Tripura, T., Hazra, B. (2019). Real time structural modal identification using recursive canonical correlation analysis and application towards online structural damage detection. Journal of Sound and Vibration, 468.

- Bernal, D. (2000): Extracting flexibility matrices from state-space realizations. In COST F3 Conference, Madrid, Spain, 1, 127–135.

- Bernal, D. (2002): Load vectors for damage localization. Journal of Engineering Mechanics, 128(1):7–14.

- Bernal, D. and Gunes, B. (2004): Flexibility based approach for damage characterization: benchmark application. Journal of Engineering Mechanics, 130(1):61–70.

- Bernal, D. (2006): Flexibility-based damage localization from stochastic realization results. Journal of Engineering Mechanics, 132(6):651–658.

- Tan, D., Ding, Z., Li, J., & Hao, H. (2023). Target-free vision-based approach for vibration measurement and damage identification of truss bridges. Smart Structures and Systems, An International Journal, 31(4), 421-436.

- Ni, Y. Q., Liu, W. Q., Rui, E. Z., Yuan, L., Chen, S. Y., & Zheng, Y. L. (2023). A novel computer vision-based vibration measurement and coarse-to-fine damage assessment method for truss bridges. Smart Structures and Systems, An International Journal, 31(4), 393-407.

- Li, J. and Qin, S. J. (2023): Applying and dissecting LSTM neural networks and regularized learning for dynamic inferential modelling. Computers and Chemical Engineering, 175.

- Loop, C. and Zhang, Z. (1999): Computing Rectifying Homographies for Stereo Vision. In: Proceedings of 1999 IEEE Computer Society Conference on Computer Vision and Pattern Recognition, 1.

- Rosten, E. and Drummond, T. (2005): Fusing points and lines for high performance tracking. In: Proceedings of IEEE International Conference on Computer Vision, Bejing, China, October.

- Harris, C. and Stephens, M. (1988): A combined corner and edge detector. In: Proceedings of the 4th Alvey Vision Conference, Manchester, UK, September.

- Lucas, B. D., Kanade, T. (1981): An iterative image registration technique with an application to stereo vision. Vancouver.

- Tomasi, C. and Detection, T. K. (1991). Tracking of point features. Int. J. Comput. Vis, 137–154.

- Tomasi, C. and Shi, J. (1993). Direction of heading from image deformations. In: Proceedings of IEEE Conference on Computer Vision and Pattern Recognition, 422–427.

- Wan, H.P. and Ren, W.X. (2016): Stochastic model updating utilizing Bayesian approach and Gaussian process model. Mech. Syst. Sig. Process., 70, 245-268.